# Environmental MonitoringReport

Semiannual Report (July–December 2021) January 2022

India: Public–Private Partnership in Madhya Pradesh Road Sector Project

MADHYA PRADESH ROAD DEVELOPMENT CORPORATION

Public Private Partnership in Madhya Pradesh Road Sector Project

(PPPMPRSP)

Under ADB Loan No – 3849- IND

SEMI ANNUAL ENVIRONMENTAL MONITORING

REPORT Project under MPRDC,

(July to 31st December 2021)

## ABBREVIATIONS

| ADB | = | Asian Development Bank |
| --- | --- | --- |
| AIDS | = | Acquired Immune Deficiency Syndrome |
| BOD | = | Biochemical Oxygen Demand |
| BDL | = | Below Detection Limit. |
| COD | = | Chemical Oxygen Demand |
| DG | = | Diesel Generator |
| EIA | = | Environmental Impact Assessment |
|  |  | Environmental Management & |
| EMMP | = | Monitoring Plan |
| EMP | = | Environmental Management Plan |
|  |  | Federation International D'Ingineure |
| FIDIC | = | Counsel |
| GRC | = | Grievance Redress Committee |
| HIV | = | Human Immunodeficiency Virus |
| HMP | = | Hot Mix Plant |
| IEE | = | Initial Environmental Examination |
| IPP | = | Indigenous People Planning |
| IRC | = | Indian Road Congress |
| Km | = | Kilometer |
| Ltd. | = | Limited |
| MOEF | = | Ministry of Environment and Forests |
|  |  | Madhya Pradesh Road Development |
| MPRDC | = | Corporation |
|  |  | National Accreditation Board for |
| NABL | = | Laboratories |
| NO2 | = | Nitrogen Dioxide |
| NOC | = | No Objection Certificate |
| pH | = | Negative log of H+ activity |
| Pkg | = | Package |
| PPE | = | Personal Protective Equipment |
| Pvt. | = | Private |
|  |  | Respirable Suspended Particulate |
| RSPM | = | Matter |
| RP | = | Resettlement Plan |
| SC | = | Supervision Consultant |
| SH | = | State Highway |
| SO2 | = | Sulfur dioxide |
| SPCB | = | State Pollution Control Board |
| SPM | = | Suspended Particulate Matter |
| TDS | = | Total Dissolved Solids |
| TSS | = | Total Suspended Solids |
| WMM | = | Wet Mix Macadam |

CONTENTS

Page

- Introduction and Project Progress during thereportingperiod	1

- Compliance on EnvironmentSafeguardsRequirements	6

  - Status of Compliance with National /State/ Local government statutory environmentalrequirements:	6

  - Status of Environmental Compliance with ADB loan covenants as Stipulated in the LoanAgreement	6

- Implementation of EnvironmentalManagementPlan	9

- Approach and Methodology for environmentalparametersmonitoring	10

- Details of Redressal committee and complaints received from public and actions taken thereoftoresolve.	11

- Action Plan for COVID PreventionandControl	11

- Follow-up ActionsandConclusions	11

Annexure-I: Status of Compliance withForestRegulations	13

Annexure-II : Status of Compliance withEnvironmentalRegulations	14

Annexure III: Checklist for the Action Taken for the Prevention of COVID 19, as per SOPMPRDC	18

- PHOTOGRAPHS	20

## Introduction and Project Progress during the reportingperiod

    - State context. Madhya Pradesh is the second largest state (area wise) and the fifth most populous state having about 72% of the population living in rural areas and is dependent upon agriculture for livelihood. An average 8% economic growth rate of state (2011 to 2018) is attributed to various industrial and infrastructural developments. The geographical conditions, resource disparity and dispersed industrial development make transport infrastructure, particularly roads, a criticalnecessity.

    - Road infrastructure in Madhya Pradesh: As of 2018, the state has a road network of about 63,637 km. About 7,806 km are national highways, 11,060 km are state highways, 22,412 km are MDRs, and the balance is other district roads and village roads. The existing road capacity is unable to cope with growing numbers of vehicle ownership (average annual vehicle registration rate of 10.3% 11.14 million vehicles in2016).

    - Some road sections are also subject to frequent flooding during monsoon season, causing accelerates road surface deterioration, congestion, limits access to goods and services, and compromises roadsafety.

    - The Government of Madhya Pradesh (GoMP), in its road sector master plan for 2013–2033, sets a vision (aligned with the agenda of the Government of India) to connect all regional and district headquarters by two lane roads and to connect all the villages by all- weather motor able roads to enhance connectivity, road safety, andquality.

    - ADB’s engagement with the road sector in Madhya Pradesh. Since 2002, the Asian DevelopmentBank (ADB) has provided the government with five loans to develop the state’s road network. Three project loans for state highways and one for MDRs have been completed, and one project loan for MDRs is ongoing (including an accident response system and an accident insurance scheme). In the five prior ADB loans, about 7,300 km of roads (11% of state roads) have beendeveloped.

    - Use of hybrid–annuity model (HAM): The first HAM project by MPRDC is a mix of the engineering, procurement, and construction (EPC) model and the build–operate–transfer (BOT) annuity model. The model was introduced by the M.P. state government in January 2016 to revive private sector investments in road infrastructure projects under Public Private Partnership arrangements with the aim to bring in private sector efficiencies for sustainable asset creation and maintenance. For this project, the government or its agency will release 60% of the total project cost during construction, which will be paid to the concessionaire in tranches linked to completion milestones. The remaining 40% is arranged by the concessionaire, usually 20%–25% (from equity), and the remaining as debt. Once the project is completed, the government will pay the concessionaire’s share of the total project cost in fixed annuity payments over the next 10years.

    - Alignment with ADB’s Strategy: The project is consistent with the operational priorities of ADB’s Strategy 2030: (i) reducing poverty and inequality, (ii) accelerating progress in gender equality, (iii) tackling climate change, and (iv) promoting rural development. The project is also consistent with the India’s partnership strategy 2018–2022 to provide inclusive access to infrastructure networks and services by strengthening PPP institutions and promoting improved contractualmodalities.

    - The Project was approved in principally by ADB on September 2018. The Project loan agreement was approved by ADB on December, 2019 and declared effective from January 2020.

    - Asian Development Bank (ADB) has approved a project loan of US$ 490 Million for the rehabilitation and upgrade of about 1600 Km of single lane state highways and major district roads in the state of Madhya Pradesh to two lane and all-weather roads. Private sector investment will be US$ 286 Million and state government are expected to contribute US$128 million. The project aims to improve transport connectivity by upgrading newly declared SH and MDRs to two lane width in public private partnership in M.P. road sector project in line with the state’s road development plan for duration year2013–2033.

    - Impact and outcome: The project is aligned to improve mobility and connectivity in India. The outcome will be improved road transport efficiency and safety within Madhya Pradesh.

    - Outputs: The project outputs will be (i) state highways upgraded to two-lane and all- weather roads, (ii) single-lane major district roads upgraded to two-lane and all-weather roads, and (iii) capacity for hybrid–annuity model implementation and road asset maintenance and managementimproved.

    - MPRDC has selected 50 roads stretches under 31 packages (packages-A has 11 contracts, packages-B has 11 contracts and packages–C has 9 contracts) with total length of 1522.69 km. All the 31 packages are scattered across the state. The details of all concerned sub-projects assigned to Independent Engineer, Bloom Companies,LLC, USA are as under in Table-1and Table-2 contains details of contractors and physical progress up to 31.12.2021.

Table 1 - Details of Package wise Independent Engineer

| Package | Name of Supervision Consultant | Contract Signing | Civil Work Contract Package No. |
| --- | --- | --- | --- |
| IE Pkg –A | Bloom Companies LLC, USA | 02.11.2021 | Pkg-1, 2, 3, 4, 5, 18, 19, 25, 35, 42, 52 |

Table 2 - Details of Civil work contracts and physical progress up to 31 Dec 2021

| SN | Packages | Project Name | Concessionaire | Length (Km) | Package Length (Km) | Cost (Cr.) | Physical Progress upto 31.12.2021 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PKG-01 | Bhabra-Kattiwada-Kheda Road | M/s ShreejiInfraspace Highways Pvt. Ltd. | 35.89 | 60.75 | 184 | 54.85% |
| 2 |  | Bhabra-Udaigarh Road |  | 24.86 |  |  |  |
| 3 | PKG-02 | Udaigarh-Bori Road | M/s ShreejiUdaigarhBalwadiInfraspace Highways Pvt. Ltd. | 15.53 | 53.16 | 164.3 | 74.83% |
| 4 |  | Tanda-Bori Road |  | 17.58 |  |  |  |
| 5 |  | Tanda-Balwadi Road |  | 20.05 |  |  |  |
| 6 | PKG-03 | Jobat-Nanpur Road | M/s NPM JobatNanpur Pathways Pvt. Ltd. | 21.87 | 21.87 | 66.6 | 91.05% |
| 7 | PKG-04 | Alirajpur-Mathwad Road | M/s Alirajput - MathwadHighways Pvt. Ltd. | 56.24 | 56.24 | 186 | 83.37% |
| 8 | PKG-05 | Badwani-Badhan-Ambapani-Sindhi-Khodar-Silawad Road | M/s Badwani-Badhan Highways Pvt. Ltd. | 17.89 | 28.91 | 127 | 97.98% |
| 9 |  | Bhavabawdi-Balkua-Lonsara-Borlay Road |  | 11.02 |  |  |  |
| 10 | PKG-18 | PipaliyaKevat-Ghatgheri(Raisen)-Ghatkheri(Raisen) to Sahajpur-Salabaru-Ghatkheri(Sagar) &Pratapgarh-Salabaru |  | 35.61 | 72.14 |  |  |
| 11 |  | Salabaru-Gatauri-Gutauri-Deori-Gorakhpur-Ghahlavan-Ghahlavan-Sadrai-Sadrai-Heerapur-Heerapur-Kaakraghat (Narmada Bridge) |  | 36.53 |  |  |  |
| 12 | PKG-19 | Maharastra Border-Kukru-Khamla-Bhaisdehi | M/s Udit Khamla Highways Pvt. Ltd. | 30.16 | 30.16 | 89.5 | 90% |
| 13 | PKG-25 | Sultaniya-Pipalkheda-Salaiya Road |  | 12.8 | 47.8 |  |  |
| 14 |  | Basoda-Gurod-Somvara-Enchda-Imaliya-Sironj Road |  | 35 |  |  |  |
| 15 | PKG-35 | Amravad-Bharkachh Road | VeljiRatnaSorathia Infra Pvt. Ltd. | 17 | 75 |  |  |
| 16 |  | Obedullaganj-Magarpunch-Ashapuri-Bhojpur Road |  | 14.6 |  |  |  |
| 17 |  | Chiklod-Raisen Road |  | 24.8 |  |  |  |
| 18 |  | Vidisha-Ahmadpur-Gadi-Gairatganj |  | 18.6 |  |  |  |
| 19 | PKG-42 | Shipra BudhiBarlai-Solsinda-Khandakhedi-Jamodi-MaharajganjBilodaNayda-PanodSanver Road |  | 28.9 | 28.9 |  |  |
| 20 | PKG-52 | Siddikganj-Hatpipaliya Road | M/s Brijgopal construction company Pvt. Ltd. | 21.24 | 48.82 | 134.99 | 5% |
| 21 |  | Bhaukhedi-Amlaha-Dhamanda Road |  | 17.85 |  |  |  |
| 22 |  | Toomda-Patania-BarkhedaSaalam Road |  | 9.73 |  |  |  |

    - Details of Organization Structure assigned for above packages are given in Table No. 3,4&5.

Table 3 - Details of Project Management Unit

| Designation | Name of the officer |
| --- | --- |
| Environment Safeguard Expert | Mr. Satyendra Bahadur IFS |
| Senior Manager Environment | Dr. Deepak Pandey |

Table 4 - Details of Civil package wise PIU Staff

| S.No. | Civil Work Package | Divisional Manager | AGM/Manager |
| --- | --- | --- | --- |
| 1 | 1 | Mr. Shyam Gupta | Mr. Dinish Patidar |
| 2 | 2 | Mr. Shyam Gupta | Mr. Dinish Patidar |
| 3 | 3 | Mr. Shyam Gupta | Mr. Dinish Patidar |
| 4 | 4 | Mr. Shyam Gupta | Mr. Dinish Patidar |
| 5 | 5 | Mr. Shyam Gupta | Mr. Kamlesh Gole |
| 6 | 19 | Mr. Praveen Nimje | Mr. R.K. Nagle |
| 7 | 52 | Mr. M.H. Rizvi | Mr. D.K. Jain |

Table 5 - Details of Supervision consultants

| Package | Name of Consulting Firm | Date of Appointment | Name of Env. Expert & Date of Mobilization | Status |
| --- | --- | --- | --- | --- |
| Pkg-1, 2, 3, 4, 5, 18, 19, 25, 35, 42, 52 | Bloom Companies LLC | 02.11.2021 | Dr Pranjan Mohanty/ 17.12.2021 | Available |

## Compliance on Environment Safeguards Requirements:

As per ADB Safeguard Policy and Environment Assessment and Review Framework (EARF) documents prepared, compliance of environment safeguard status has been evaluated in two sections given asunder:

## Status of Compliance with National /State/ Local government statutory environmental requirements:

- MPRDC has obtained/is obtaining permissions required for working in the forest area. MPRDC and IE have instructed the contractors not to do the construction work in the forest stretch of the road for which permission from the forest department is yet to be received. Permission for cutting trees is also being obtained where tree cutting is unavoidable. MPRDC and has regularly asked Civil Work Contractors to obtain all the permission required under the existing statutory environmental regulations and submit the complete details regarding the status of clearances/approvals/ consents applied for/obtained. MPRDC through PIU is also periodically writing to contractors to obtain Environmental clearances for Quarry sites and Borrow areas from DEIAA/SEIAA/MoEF&CC (as applicable). Contractors are also regularly being reminded through IE for obtaining pending permissions/consents (if any) such as consent to establishment and operation for crusher, batching plant, DG sets, Quarry sites and Borrow areas etc. at their respective sites from State Pollution Control Board. The status of compliance with Forest Regulations and Status of Compliance with Environment Regulations for the reporting period is given below in Annexure 1&2.

## Status of Environmental Compliance with ADB loan covenants as Stipulated in the Loan Agreement.

Table 6 - Compliance Status with ADB Loan Covenants

| Ref. | Particulars | Reference | Status of Compliances |
| --- | --- | --- | --- |
| 1 | Conditions for award of contracts; commencement of Works (Schedule 4, para 6) |  |  |  |
|  | The Borrower shall ensure, or cause the EA to ensure, that no Works contract is awarded which involve environmental impact for a Subprojects until the EA has: |  |  |  |
|  | (a) prepared, submitted and obtained the final approval of the relevant IEE from ADB and the relevant forest and environmental clearances from statutory authorities of the borrower and the state as applicable; and | Schedule 4, para 6(a) | being complied |  |
|  | (b) Incorporate relevant provisions from the respective EMP into the related work contract | Schedule 4, para 6(b) | being complied |  |
| 2 | Commencement of Works (Schedule 4, para 6) |  |  |  |
|  | The Borrower shall ensure, or cause the MPRDC to ensure, that no commencement of Works is allowed under any Works contract under any Subproject which involves environmental impacts and requires environmental clearances, until MPRDC has obtained the final approval of the IEE from ADB, and environmental clearance including approval of the environmental assessment report, from the State Environmental Impact Assessment Authority. |  | Being followed |  |
| 3 | Environment (Schedule 5, para 3) |  |  |  |
|  | The Borrower shall ensure or cause the EA to ensure that the preparation, design construction, implementation, operation and decommissioning of the Project facility comply with- | Schedule 4, para 10 |  |  |
|  | (i) All applicable laws and regulations of the borrower and the State relating to Environment, health and safety. | Schedule 4, para 10 (a) | Being Complied and periodical monitoring is also being done. |  |
|  | (ii) The Environmental Safeguards | Schedule 4, para 10 (b) | Being Complied and periodical monitoring is also being done. |  |
|  | (iii) The EARF and | Schedule 4, para 10(c) | Being Complied and periodical monitoring is also being done. |  |
|  | (iv) All measures and requirements set forth in the relevant IEE and EMP, and any corrective or preventative actions set forth in a Safeguards Monitoring Report, | Schedule 4, para 10(d) | Being Complied and periodical monitoring is being done. |  |
| 4 | Human and Financial Resources to Implement Safeguards Requirements (Schedule 5, para 6) |  |  |  |
|  | (a) The Borrower shall make available or cause the EA to ensure that all necessary budgetary and human resources to fully implement each of the respective EMP, the RP and the IPP (as applicable) are made available on timely basis. | Schedule 4, para 15 | Being compiled |  |
| 5 | Safeguard – Related provisions in Bidding Documents and Works Contract (Schedule 5, para 7) |  |  |  |
|  | The Borrower shall ensure or cause the EA to ensure that all bidding documents and contracts for Works contain provisions that require contractors to: | Schedule 4, para 16 |  |  |
|  | (a) Comply with the measures and requirement relevant to the Contractor set forth in IEE, the EMP, and the RP and the IPP (as applicable), and any corrective or preventative actions set out in a Safeguards Monitoring Report; | Schedule 4, para 16(a) | being complied |  |
|  | (b) Make available a budget for all such environmental and social measures; | Schedule 4, para 16(b) | being complied |  |
|  | (c) Provide the EA with a written notice of any unanticipated environmental, resettlement or indigenous peoples risks if any, or impacts that arise during construction, implementation or operation of the project that were not considered in the IEE, the EMP and the RP or the IPP (as applicable); | Schedule 4, para 16(c) | being complied |  |
|  | (d)adequately record the condition of roads, land and other infrastructure prior to starting to transport materials and constructions and; | Schedule 4, para 16(d) | being complied |  |
|  | (e) Fully reinstate pathways, other local infrastructure, and agricultural land to at least their pre-project condition upon the completion of construction. | Schedule 4, para 16(e) | being complied |  |
| 6 | Safeguards Monitoring and Reporting (Schedule 5, para 8) |  |  |  |
|  | The Borrower shall do or causes the EA to do the following: | Schedule 4, para 17 |  |  |
|  | (a) Submit semiannual Safeguards Monitoring Reports for Environmental Safeguards to ADB no later than 3 months after the end of the respective reporting period to which each relates and disclose relevant information from such reports to affected persons promptly upon submission; | Schedule 4, para 17(a) | Being complied |  |
|  | (b) If any unanticipated environmental and/or social risk and impacts arise during construction, implementation or operation of the project that were not considered in the IEE, the EMP, and the RP or the IPP as applicable, promptly inform ADB of the occurance of such risk or impact with detailed description of the event and proposed corrective action plan; and | Schedule 4, para 17(b) | Being complied |  |
|  | Throughout the term of the project engage qualified and experienced external expert or qualified non-government organization under the selection process and terms of references applicable to adb to verify information produced through the project monitoring process and facilitate the carrying out of any verification by such external expert | Schedule 4, para 17(c) | Being complied |  |
|  | (c) Report any breach of compliance with the measures the requirements set forth in the EARF, IEE the RF, RP and the IPPF, IPP (if any), promptly after becoming aware of the breach. | Schedule 4, para 17(d) | Being complied |  |
| 7 | Prohibited List of Investment (Schedule 5, para 9) |  |  |  |
|  | The borrower shall ensure, or cause EA to ensure, that no proceeds of the loan are used to finance any activity included in the list of prohibited investment activities provided in Appendix 5 of ADB’s SPS | Schedule 4, para 18 | Being complied |  |
| 8 | Labour Standards (Schedule 5, para 10) |  |  |  |
|  | The Borrower, shall ensure or cause the EA to ensure the works contracts under the Project follow all applicable labor laws of the Borrower and the State and that these further include provisions to the effect that Contractors; | Schedule 4, para 19 | Being complied |  |
|  | (i) Carryout HIV/AIDS awareness programmes for labour and disseminate information at worksites on risk of sexually transmitted diseases and HIV/AIDS as part of health and safety measures for those employed during construction; and | Schedule 4, para 19(a) | Being complied |  |
|  | (ii) Follow and implement all statutory provisions on labour (including not employing or using children and labour, equal pay for equal work), health safety, welfare, sanitation, and working conditions. Such contracts shall also include clauses for termination in case of any breach of the stated provisions by the Contractors. | Schedule 4, para 19 (b) | Being complied |  |

## Implementation of Environmental Management Plan:

The IEE duly showed that the environmental impact would mostly occur during construction due to clearing for widening the carriageway, construction work related to compaction of the embankment and cut and fill, as well as the quarrying and borrowing to support construction activity. The environment category for all roads projects is “B”. During implementation, MPRDC implemented the overall mitigation measures. Environmental and Social Cell of MPRDC& IE is responsible for monitoring of EMP implementation. MPRDC and Civil works contractors obtained/obtaining all permissions required from related government agencies, as per construction applicable, statutory environmental regulations to enable them to carry out without violating country rules and regulations. MPRDC through its Environmental Safeguard Expert and the IE and PIU staff are closely monitoring the implementation of environmental issues of all packages. Regular site visits and review meetings are being held with the nominated Environmental Officers of Contractor’s staff of each project. In addition to this, for effective day to day supervision of implementation of environmental aspects with the designated site staff (site engineer) to look after these aspects. The contractors have appointed /nominated safety officer at their construction sites to take care of safety aspects of construction sites. The list of nominated Environment, Health &Safety officers package wise is given in Table:7 below. Structure of Institutional setup to monitor EMP implementation on each project is being shown below:

Environmental Safeguard Expert

Table 7 - Details of Contractor’s Environment, Health & Safety officer

| Package No. | Name of Road | Contractor Name | Name of EHS officer of contractor |
| --- | --- | --- | --- |
| 1 | Bhabra-Kattiwada-Kheda road | M/s ShreejiKathiwadaBabra- udaigarh Highways Pvt. Ltd. | Neeraj Vishwakarma |
|  | Bhabra-Udaigarh road |  |  |
| 2 | Udaigarh-Bori road |  |  |
|  | Tanda-Bori road |  |  |
|  | Tanda-Balwadi road |  |  |
| 3 | Jobat-Nanpur road | M/s NPM JobatNanpur Pathways Pvt. Ltd. | Sanjay Kumar Rai |
| 4 | Alirajpur-Mathwad road | M/s Alirajpur Mathwad Pvt. Ltd. | Saurabh Yadav |
| 5 | Badwani-Badhan-Ambapani-SindhiKhodar-Silawad road | M/s Badwani-Badhan Highways Pvt. Ltd | Saurabh Yadav |
|  | Dhavabawdi-Balkua-Lonsara-Borlay road |  |  |
| 52 | Siddikganj-Hatpipaliya Road | M/s BG MP Highways Pvt. Ltd | Pradeep Prajapati |
|  | Bhaukhedi-Amlaha-Dhamanda Road |  |  |
|  | Toomda-Patania-BarkhedaSaalam road |  |  |

The IE submit Monthly/Quarterly progress report to MPRDC that includes information on implementation of environmental issues. The environmental information, captured in these reports includes environmental monitoring status of construction sites, pollution control measure being adopted, and measures for workers safety at construction site and camp site, safety of road user/public etc. The PIU has assessed the implementation of EMP during the reporting period as satisfactory. However, safety arrangements, traffic management and usage of personal protective equipment (PPE) by the construction workers in all packages need to be improvedfurther.

The IE has instructed the contractor to organize HIV/AIDS awareness and health check-up campaigns at project sites with the aim to reduce transmission risk of HIV infection among the construction workers and population living along the projectroads.The HIV/AIDS

Camp and health check-up camp details are given in Table 8.

Table 8 - Details of HIV/AIDS Awareness program

| Package | Road Name | Date of awareness program Location | Location |
| --- | --- | --- | --- |
| 1 | Bhabra-Kattiwada-Kheda road | 17.09.2021 | At Camp |
|  | Bhabra-Udaigarh road |  |  |
| 2 | Udaigarh-Bori road | 30.08.2021 | Kunalvasa village |
|  | Tanda-Bori road |  |  |
|  | Tanda-Balwadi road |  |  |
| 3 | Jobat-Nanpur road | 10.12.2021 | Khattali village camp. |
| 4 | Alirajpur-Mathwad road | 06.09.2021& 8.12.2021 | AmbajaVillage Camp |
| 5 | Badwani-Badhan-Ambapani-SindhiKhodar-Silawad road | 07.09.2021 | Sindhi khodri Village Camp |
|  | Dhavabawdi-Balkua-Lonsara-Borlay road |  |  |
| 52 | Siddikganj-Hatpipaliya Road | HIV/AIDS Camp was not conducted during the reporting period | NIL |
|  | Bhaukhedi-Amlaha-Dhamanda Road |  |  |
|  | Toomda-Patania-BarkhedaSaalam road |  |  |

Table 9 - Details of Public Consultations conducted during the Reporting period

| S. No | Package |  | Name of Project Road |  |  | Date | Location/Chainage | No of Participants | Issues Discussed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Package |  |  |  |  |  |  |  |  |  |
| 1. | 1 |  | Bhabra-Kattiwada-Kheda road |  |  |  | NOT AVAILABLE |  |  |
|  |  |  | Bhabra-Udaigarh road |  |  |  |  |  |  |
| 2. | 2 |  | Udaigarh-Bori road |  |  |  | NOT AVAILABLE |  |  |
|  |  |  | Tanda-Bori road |  |  |  | NOT AVAILABLE |  |  |
|  |  |  | Tanda-Balwadi road |  |  |  | NOT AVAILABLE |  |  |
| 3 | 3 |  | Jobat-Nanpur road |  |  |  | NOT AVAILABLE |  |  |
| 4 | 4 |  | Alirajpur-Mathwad road |  |  | 06-09-2021. 09-12-2021. | Ambaja Village Ambaja Village | 17 18 | Safety awareness among the participants was discussed. |
| 5 | 5 |  | Badwani-Badhan-Ambapani-SindhiKhodar-Silawad road |  |  | 07-09-2021 | Silawad | 13 | Road safety measures and safety awareness among the participants was discussed. |
|  |  |  | Dhavabawdi-Balkua-Lonsara-Borlay road |  |  |  |  |  |  |
| 6 | 52 |  | Siddikganj-Hatpipaliya Road |  |  |  | Public consultation not done in the reporting period. |  |  |
|  |  |  | Bhaukhedi-Amlaha-Dhamanda Road |  |  |  | Bhaukhedi & Dhamanda | 14 11 | Breaker near School Starting &end. Proper road to be completed soon as water logging is reported on the Road. |
|  |  |  | Toomda-Patania-BarkhedaSaalam road |  |  |  | Public consultation not done in the reporting period. |  |  |

## Approach and Methodology for environmental parameters monitoring

The IE &PIU has instructed the civil work contractors to ensure the monitoring of ambient air quality, water quality and noise level as per the agreed environmental monitoring plans. As per” Environmental Monitoring Plan,” The contractor shall carry out periodic monitoring of Environmental attributes such as Air quality, Water (Surface water and ground water) Quality and Noise quality at the predefined locations through hired external monitoring agency which must be accredited with NABL/SPCB/ CPCB. The sampling location for these components based on the monitoring stations with reference to IEE report and environmental sensitive locations like school, hospital, construction sites, pollution sources, discharge points, camp sites, forest area (If any) etc. have been identified. The details on environmental components, parameters, methodology, duration of monitoring and instruments are described in following table.

| Env. Component | Parameters ( As given in IEE) | Method of Monitoring | Duration/Frequency of Monitoring | Instrument used |
| --- | --- | --- | --- | --- |
| Air | PM2.5, PM10, SO2, NOX, CO. | High volume sampler to be located 50m from the plant in the Downwind direction. Use method specified by CPCB for analysis | Thrice in a year except monsoon | High volume sampler |
| Water | pH, Conductivity, TDS, TSS, turbidity, Total alkalinity, Total Hardness, Chloride, Iron, BOD, COD, Oil & grease, etc. | Grab sample collected from source and Analyse as per Standard Methods for Examination of Water and Wastewater | Thrice in a year except monsoon | - |
| Noise | Equivalent Noise levels on dB (A) scale for day and night | By using Noise meter Readings to be taken at 15 seconds interval for 15 minutes every hour and then averaged | Thrice in a year except monsoon | Noise Meter |

Table 10 - ADB Approved Environmental Monitoring Plan

Environment Monitoring (Ambient air, surface water, ground water and Noise) Status and findings:-

During this reporting period, monitoring of components such as ambient air quality, water quality and noise level was carried out by the civil work contractors of Package 1,2, 3 , 4& 5

under supervision Package-A, Bhopal, Package 6. Details are provided in the Table-11,12,

13 &14. . The contractors have been instructed to do the monitoring work as per agreed environmental monitoring plan where the monitoring activities were still not taken up.

Table 11: Status of Environmental Monitoring during Reporting Period

| Name of the Sub-Project | Monitoring Location | Parameter | Month* | Observations |
| --- | --- | --- | --- | --- |
| (Package-1, Bhopal) |  |  |  |  |
| Package-1 | Air Sampling:- - Labour Camp - Stone Crusher - Camp Office Noise Sampling:- - Labour Camp - Stone Crusher - Road side village Water Sampling:- - Labour Camp - Stone Crusher - Road side village | Air- PM10, PM2.5, SO2, NOX, CO Water:- Surface Water - pH, TDS, TSS, COD, BOD, Oil and Grease, Total Coliform (MPN Count). Ground water: Parameter for drinking water as per IS:10500, Noise: Equivalent Noise level | 19.12.21 19.12.21 19.12.21 | All the Parameters (for Air, Water, and Noise) are found within the permissible limits. |
| Package-2 | Air Sampling:- - Labour Camp - Stone Crusher - Camp Office Noise Sampling:- - Labour Camp - Stone Crusher - Road side village Water Sampling:- - Labour Camp - Stone Crusher - Road side village | Air- PM10, PM2.5, SO2, NOX, CO Water:- Surface Water - pH, TDS, TSS, COD, BOD, Oil and Grease, Total Coliform (MPN Count). Ground water: Parameter for drinking water as per IS:10500, Noise: Equivalent Noise level | 17.12.21 17.12.21 17.12.21 | All the Parameters (for Air, Water, and Noise) are found within the permissible limits. |
| Package-3 | Air Sampling:- - Labour Camp - Stone Crusher - Camp Office Noise Sampling:- - Labour Camp - Stone Crusher - Road side village Water Sampling:- 1. Labour Camp | Air- PM10, PM2.5, SO2, NOX, CO Water:- Surface Water - pH, TDS, TSS, COD, BOD, Oil and Grease, Total Coliform (MPN Count). Ground water: Parameter for | 20.12.2021 22.12.2021 | All the Parameters (for Air, Water, and Noise) are found within the permissible limits. |

| Name of the Sub-Project | Monitoring Location | Parameter | Month* | Observations |
| --- | --- | --- | --- | --- |
|  | - Stone Crusher - Road side village | drinking water as per IS:10500, Noise: Equivalent Noise level | 24.12.2021 |  |
| Package-4 | Air Sampling:- - Labour Camp - Stone Crusher - Camp Office Noise Sampling:- - Labour Camp - Stone Crusher - Road side village Water Sampling:- - Labour Camp - Stone Crusher - Road side village | Air- PM10, PM2.5, SO2, NOX, CO Water:- Surface Water - pH, TDS, TSS, COD, BOD, Oil and Grease, Total Coliform (MPN Count). Ground water: Parameter for drinking water as per IS:10500, Noise: Equivalent Noise level | 5.11.2021 5.11.2021 5.11.2021 | All the Parameters for Air quality, water quality and Noise quality are found within permissible limits. |
| Package-5 | Air Sampling:- - Labour Camp - Stone Crusher - Camp Office Noise Sampling:- - Labour Camp - Stone Crusher - Road side village Water Sampling:- - Labour Camp - Stone Crusher - Road side village | Air- PM10, PM2.5, SO2, NOX, CO Water:- Surface Water - pH, TDS, TSS, COD, BOD, Oil and Grease, Total Coliform (MPN Count). Ground water: Parameter for drinking water as per IS:10500, Noise: Equivalent Noise level | 5.11.2021 5.11.2021 5.11.2021 | All the Parameters for Air quality, water quality and Noise quality are found within permissible limits. . |

Table 12 : Ambient Air Quality Monitoring Result.

| Parameters | PM10 (µg/m3) | PM2.5 (µg/m3) | SO2 (µg/m3) | NOx (µg/m3) | CO (µg/m3 ) |
| --- | --- | --- | --- | --- | --- |
| Package 1 | 72 | 26 | 10 | 22 | 1000 |
| Package 2 | 75 | 27 | 11 | 25 | 900 |
| Package 3 |  | 44.7 | 25.4 | 16.5 | 250 |
| Package 4 | 96.63 | 44.62 | 38.88 | 26.39 | 710.2 |
| Package 5 | 96.63 | 44.02 | 38.88 | 26.39 | 710.2 |
| Package 52 | Not available | Not available | Not available | Not available | Not available |

Table 13 : Ambient Noise Level Monitoring Result.

| Parameters | Day Time dB(A) |
| --- | --- |
| Locations | Leq |
| Package1 | 73.4 |
| Package 2 | 75 |
| Package 3 | 75 |
| Package 4 | 65 |
| Package 5 | 65 |
| Package 52 | Not available |

TABLE 14: Ground/Surface Quality Water Test Results

| Parameters | pH | TDS (mg/l) | Total Hardness as CaCO3 (mg/l) | Iron as Fe (mg/l) | Chloride as Cl (mg/l) | Total Coliform(MPN/100 ml) |
| --- | --- | --- | --- | --- | --- | --- |
| Package1 | 7.66 | 395 | 329 | N.D | 261 | Absent |
| Package 2 | 7.88 | 293 | 198 | N.D | 34.9 | 18 |
| Package3 | 7.4 | 450 | 400 |  | 75 | Absent |
| Package 4 | 7.62 | 487 | 195 | 0.106 | 33.7 | Absent |
| Package 5 | 7.62 | 487 | 195 | 0.106 | 33.7 | Absent |
| Package 52 |  |  |  |  |  |  |

## Details of Redressal committee and complaints received from public and actions taken thereof toresolve.

The GRC (Grievance Redressal Committee) for both state (MPRDC) and Divisional level (PIU) have been constituted. In addition, a register of grievances is also placed in construction camp to register people’s grievances, but the grievances related to environmental issues were not received till the end of reportingperiod.

## Action Plan for COVID Prevention andControl

COVID-19 illness tuned to a pandemic covering major part of the global engraved roots in India and Madhya Pradesh in the month of March 2020. However, ADB surveillance to pandemic was started with the world health organization’s guidelines that were evident from the correspondences of ADB officials. Then after, MPRDC officials have formulated standard operating procedures to resume the work related to onsite road construction as well as official works. The SOP was well informed to ADB before actual implementation. All the PIU & Staff and contractor were asked to develop the action plan to be implemented during the normal working hours. MPRDC ensured that the action plan submitted by the consultant and contractor are as per national and international guidelines to provide utmost safety against the spread of COVID 19 diseases. Reporting of safety measures implemented to combat COVID related diseases and compliance of protective measures enlisted in SOP are made mandatory in monthly progress reports as well as during the official meetings. A sample of COVID prevention action check list is attached (Annexure III). Also attached is the status of action taken from civil work contractors.

## Follow-up Actions and Conclusions

The Contractors of all packages have been instructed to strictly adhere to the relevant mitigation measures as per EMP during the execution of all activities at construction sites. Following are the areas for scope of improvements by the contractor during the course of construction works:

  - Permission/Renewal of statutory consents and permissions from concerning Department.

  - Regular Environmental Monitoring for Air Quality, Noise level and water quality is required as per Environment Monitoring Plan.

  - Providing Personal protective equipment to all officers and workers in construction site and campsite.

Regular monitoring of compliance of COVID prevention action plan and related SOP in construction site, labour camps, work offices etc.

## Annexure 1

PPPMPRSP

Status of Compliance with Forest Regulations (July 2021 – Dec 2021)

| Contract Package | Name of ProjectRoad | District | Length of Project Road Km | Length of package Road km | Status of TreeCutting | Status of Working Permission |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | Bhabra – Katthiwada – Kheda Road & | Alirajpur | 35.891 | 60.752 | 359 trees to be cut. | Obtained DM,Alirajpur vide letter No.004/A-62/2019-20 dated 12.12.2019. |
|  | BhabraUdaigarh | Alirajpur | 24.860 |  | Obtained | Obtained DM,Alirajpur vide letter No.004/A-62/2019-20 dated 12.12.2019 |
| P2 | Udaigarh – Bori | Alirajpur | 18.950 | 54.650 | Obtained | Obtained |
|  | Tanda – Bori | Dhar | 14.700 |  | Obtained | Obtained. |
|  | Tanda–Balwadi | Dhar | 21.000 |  | Obtained | Obtained |
| P3 | Jobat – Nanpur Road | Alirajpur | 21.600 | 21.600 | Not Required | NIL |
| P4 | Alirajpur – Mathwad Road | Alirajpur | 57.000 | 57.000 | DM, Alirajpur letter no 588/2018 dated 17/12/2018.Ordered for “AS AND WHERE BASIS” to be maintained. | Obtained. DM, Alirajpur letter no 588/2018 dated 17/12/2018.Ordered for “AS AND WHERE BASIS” to be maintained |
| P5 | Badwani– Badhan– Ambapani– Sindhi– Khodar– Silawad | Badwani | 14.400 | 25.200 | Obtained | No information Received. |
|  | Dhavabawdi– Balkua– Lonsara–Borlay | Badwani | 10.800 |  | Obtained | No information Received. |
| P52 | Toomda – Patania – BarkhedaSaalam Road | Bhopal | 9.730 | 48.820 | No information Received. | No information Received. |
|  | Siddikaganj – Hatpipaliya Road | Sehore | 21.240 |  | No information Received. | No information Received. |
|  | BhaukhediAmlaha – Dhamanda Road | Sehore | 17.850 |  | 145 trees to be cut as per DM,Sehore order. | DM SEHORE letter No 01/A-62/2021-22 dated 10/12/2021 for tree cutting. |

Annexure 2: Status of Compliance with Environmental Regulations (Jul-2021 – Dec 2021)

| S.No/ Pkg | Name of the Sub-Project Road | Env. Clearance for the Project from MoEF/ SEIAA | Environmental Consents/ Clearances to be obtained by Contractor |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Crusher (Ref Agreement clause No-4.18) | Batching Plant (Ref Agreement clause No-4.18) | Diesel Generator Set (Ref Agre | Pollution Under Control (PUC) certificates for vehicle | Permission obtained from other regulators (Ref. Agreement Clause-4.18) |
| Pkg-1 | Bhabra – Katthiwada – Kheda Road & Bhabra Udaigarh Total Length: 60.752kms. Contractor- M/s Shri ji KatthiwadaBhabhraUdaygarh Highway P. L. | As per MoEF amendment in EIA notification 2006, EC is not required | 1. Crusher Plant: CTE – Obtained. Consent no: CTE 86639 dated 30/03/2021. CTO – Obtained . Consent no: CTE 86639 dated 30/03/2021. 2. RMC Plant and DG Set – Obtained . Consent no: CTE 86639 dated 30/03/2021. CTO – Obtained. Consent no: CTE 86639 dated 30/03/2021. Vehicles PUC certificate: Obtained. |  |  |  | Labour License: Obtained. Valid till 31.12.2021 SEIAA Permission For stone Quarry – Obtained. Vide letter No 1811/SEIAA/13.9.2021 SEIAA Permission for borrow area – Obtained Sand Source: Obtained Workmen’s Insurance Policy:-obtained . valid till 07.01.2023 CAR Policy: Obtained.valid till 22/02/2023. |
| Pkg – 2 | Udaigarh – Bori Tanda – Bori Tanda–Balwadi Total Length: 54.650kms. Contractor - M/s ShrijiUdaygarhBalwadi Highway P. L | As per MoEF amendment in EIA notification 2006, EC is not required | 3. Crusher Plant: CTE – Obtained . Consent no: CTE 86639 dated 30/03/2021. CTO – Obtained . Consent no: CTE 86639 dated 30/03/2021. 4. RMC Plant and DG Set – Obtained. Consent no: CTE 86639 dated 30/03/2021. CTO – Obtained. Consent no: CTE 86639 dated 30/03/2021. Vehicles PUC certificate: Obtained |  |  |  | Labour License: Obtained. Valid till 31.12.2021.. SEIAA Permission For stone Quarry – Obtained. Vide letter No 1813/SEIAA/13.9.2021 SEIAA Permission for borrow area – Obtained Sand Source: Obtained Workmen’s Insurance Policy:- obtained .valid till 07.01.2023 CAR Policy: Obtained. Valid till 23/02/2023 |
| Pkg - 3 | Jobat – Nanpur Road Total Length 21.600 kms. Contractor: M/s NPM JobatNanpurPahtway sP.L.. | As per MoEF amendment in EIA notification 2006, EC is not required | - Crusher Plant: CTE – Obtained.PCB ID :21219 consent obtained Valid till 15/07/2024 - CTO – Obtained. PCB ID :21219 consent obtained Valid till 15/07/2024 - 6. RMC Plant and DG Set – Obtained. PCB ID :21219 consent obtained Valid till 15/07/2024 - CTO – Obtained. PCB ID :21219 consent obtained Valid till 15/07/2024 Vehicles PUC certificate: Obtained valid till 10.5.2022. |  |  |  | Labour License:Obtained valid till 31.12.2021 SEIAA/DEIAA Permission For stone Quarry – Obtained. Vide letter No.29/DEIAA/ALI/2018. SEIAA/DEIAA Permission for borrow area – Obtained.DM letter no 24/6/2017 Sand Source: Obtained Workmen’s Insurance Policy:-obtained CAR Policy: Obtained |
| Pkg - 4 | Alirajpur – Mathwad Road Total Length 57.00 kms. Contractor: M/s Alirajpur – Mathwad Highways P. L. | As per MoEF amendment in EIA notification 2006, EC is not required | 7. Crusher Plant: CTE – consent establishment no 14978,27/02/2021 Valid till 28.02.2022 CTO – 8. RMC Plant – Obtained Obtained, consent no CTE-85664 & outward No 14977/27.02.2021 valid till 31.01.2022. CTO - Obtained 9. DG Set CTE/CTO – Obtained, consent no CTE-85664 & outward No 14977/27.02.2021 valid till 31.01.2022. Vehicles PUC certificate: Obtained , valid till 19.06.2022. |  |  |  | Labour License: Obtained.outward no.25863,dated 09.01.2020 valid till 20.01.2022 SEIAA Permission For stone Quarry – Obtained. outward no 14921, 05/02/2021 valid till 31.01.2022. SEIAA Permission for borrow area – Obtained.approved with 2380/ADB-vi /MPRDC/PKG-4/2020-2021 dated 11.12.2020. Sand Source: Obtained. Workmen’s Insurance Policy:- obtained CAR Policy: Obtained. Valid till 25.09.2022. |
| Pkg - 5 | Badwani– Badhan– Ambapani– Sindhi– Khodar– Silawad Dhavabawdi– Balkua– Lonsara–Borlay Total Length 25.200 kms. Contractor: M/s Badwani - Bandhan Highways P. L. | As per MoEF amendment in EIA notification 2006, EC is not required | 10. Crusher Plant: CTE – Obtained.outward No.21167,04/05/2021 consent no. 87340. CTO – Obtained. Consent no. 84728 & outward No. 20540/02.02.2021 Valid till 31.12.2025. 11. RMC Plant – Obtained Consent no. 84728 & outward No. 20540/02.02.2021 Valid till 31.12.2025. CTO - Obtained Consent no. 84728 & outward No. 20540/02.02.2021 Valid till 31.12.2025. 12. DG Set CTE/CTO – Obtained Consent no. 84728 & outward No. 20540/02.02.2021 Valid till 31.12.2025. Vehicles PUC certificate: Obtained . Valid till April 2022. |  |  |  | Labour License: Obtained .outward no.44642 dated 09.02.2021 valid till 31.12.2022. SEIAA Permission For stone Quarry – Obtained.vide outward no 21166 dated 04/05/2021 . SEIAA Permission for borrow area – Obtained. Vide letter no 2370/ADB-VI/MPRDC/PKG-5/2020 dated 9.12.2020. Sand Source: Obtained Workmen’s Insurance Policy:- obtained CAR Policy: Obtained .Valid till 25.09.2022. |
| Pkg-19 | Bhaisdehi – KukruKhamla Maharashtra Border Road Total Length: 30.00kms. Contractor M/s UditKhamla Highways P. L. | As per MoEF amendment in EIA notification 2006, EC is not required | 13. Crusher Plant: CTE – Obtained CTO – Obtained 14. RMC Plant and DG Set – Obtained CTO - Obtained Vehicles PUC certificate: Obtained |  |  |  | Labour License: Obtained SEIAA Permission For stone Quarry – Obtained SEIAA Permission for borrow area – Obtained Sand Source: Obtained Workmen’s Insurance Policy:-obtained CAR Policy: Obtained |
| Pkg – 52 | Toomda – Patania – BarkhedaSaalam Road, Siddikaganj – Hatpipaliya Road, BhaukhediAmlaha – Dhamanda Road Total Length 48.820 kms. Contractor: M/s BG MP Highways P. L. | As per MoEF amendment in EIA notification 2006, EC is not required | 15. Crusher Plant: CTE – NotObtained CTO – NotObtained 14. RMC Plant and DG Set – Not Obtained CTO –Not Obtained Vehicles PUC certificate: Obtained.Valid till June 2022. |  |  |  | CAR Policy: Obtained.valid till 30.03.2023. Borrow area- Applied Sand Source: Obtained Permission for Stone Quarry : Obtained.vide letter No. 6967/SEIAA/15 dated 30.10.2015 Labour licence: Applied. |

PHOTOGRAPHS Pkg- 1& 2

Health Awareness Programme			Health Awareness Programme

Covid - 19 Camp						Covid - 19 Camp

## PHOTOGRAPHS Pkg-3

## Camp Site Photographs				Mess Photographs

Camp Site					Environment Testing

HIV/AIDS Camp					Health Camp

COVID Camp						Women Health Camp

Compliant Register

## PHOTOGRAPHS Pkg-4

HIV/AIDS Camp						HIV/AIDS Camp

Public Consultation					Public Consultation

Public Consultation					Public Consultation

PHOTOGRAPHS Pkg-5

HIV/AIDS Camp						HIV/AIDS Camp

Public Consultation					Public Consultation

Pkg-52

Public Consultation

Pkg-52 Public Consultation

| Package – 03 Health Camp |  |
| --- | --- |

Package -52 Public Consultation

Package -52 Public Consultation

DETAILS OF TREE PLANTATION

| Pkg. | Name of Road | Compulsory Afforestation (Nos) | Compensatory Afforestation 10 Times of tree cut (Nos) | Target for this rainy season | Plantation Completed | Status of Forest Proposal /Permission |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.Bhabra-Kattiwada-Kheda road 2. Bhabra-Udaigarh road | 1-4307 2-2983 | 590 | - 2500 - 1500 | Completed -7290 | IE/DM to update |
| 2 | 1.Udaigarh-Bori Road 2. Tanda-Bori Road 3. Tanda-Balwad Road | 1-1900 2-2110 3-2406 | - 1090 - 240 | - 1000 - 1500 - 1500 | Completed -2487 | IE/DM to update |
| 3 | Jobat-Nanpur road | 2625 | 880 | 2625 | Completed -2625 | IE/DM to update |
| 4 | Alirajpur-Mathwad road | 6836 | 2170 | 5000 | Completed -7650 | IE/DM to update |
| 5 | 1.Badwani-Badhan-Ambapani-Sindhi-Khodar-Silawad road 2.Dhavabawdi-Balkua-Lonsara-Borlay road | 1-2146 2-1322 | 2 - 4250 | 1-1500 2-1000 | Completed -4800 | IE/DM to update |
| 19 | Maharastra Border-Kukru-Khamla-Bhaisdehi | 3620 | IE/DM to update | 3620 | Completed -2600 | .IE/DM to update |
| 52 | 1.Siddikganj-Hatpipaliya Road 2.Bhaukhedi-Amlaha-Dhamanda Road 3.Toomda-Patania-Barkheda Saalam road | 1-2548 2-2142 3-1168 | IE/DM to update | 1-2548 2-2142 3-1168 | - | IE/DM to update |

Pkg-2 Environmental Test Report

| Air Sample Test Report | Air Sample Test Report |
| --- | --- |

| Noise Sample Test Report | Noise Sample Test Report |
| --- | --- |

| Water Sample Test Report | Soil Sample Test Report |
| --- | --- |

| Package – 01 Environmental Test Report |  |
| --- | --- |

| Package- 01 Environmental Test Report |  |
| --- | --- |

Pkg- 4

| Air Sample Test Report | Air Sample Test Report |
| --- | --- |

| Noise Sample Test Report | Noise Sample Test Report |
| --- | --- |

| Water Sample Test Report | Water Sample Test Report |
| --- | --- |