Chapter 9:

Modern analytical techniques used for the identification of bioactive molecules.

Deepak Kumar1, Sudipto Debnath1, Kalyan Hazra1, Jyoti Dahiya1

1Central Ayurveda Research Institute, CCRAS, 4-CN Block, Salt Lake Sector V, Bidhannagar, Kolkata, India 700091

8.1 Introduction

Plants serve as a repository for thousands of structurally diverse bioactive molecules. The abundant secondary metabolites present in plant show interesting bioactivities because of their diverse range of structural arrangements and characteristics. Natural compounds contained in various portions of plants are an attractive subject of research because of several bioactivities of these molecules such as antioxidant, antiinflammatory, immunomodulatory, and so on. The unique properties of bioactive molecules have compelled researchers to characterize these systems both by experimental and theoretical methods. The separation of bioactive molecules from plants, as well as their qualitative and quantitative estimation are crucial steps in the discovery of new biomolecules and can be utilized as a lead molecule to synthesise increased efficacious molecules. However, we all know that natural products are complex mixtures made up of numerous chemical constituents and have synergistic effects [1]. Therefore, it is challenging to assess and pinpoint the bioactive components which might be responsible for their efficacy [2]. There is a need for the creation of trustworthy techniques for natural product’s bioactive components to be screened for potential novel therapeutic leads [3]. Building natural product libraries will benefit greatly from the discovery of novel bioactive molecules [4]. Because different plant materials include a variety of bioactive compounds, practical research demands the selection and optimization of each step of the entire analytical procedure. In plants, bioactive substances can be classified into a number of types based on their metabolic processes: glycosides, phenolic, flavonoids, stilbenes, anthocyanins, tannins, terpenoids, lignans, resins, alkaloids, proteins and peptides etc. [5, 6]. All of these classes have unique chemical compositions and physical-chemical characteristics, necessitating the use of various analytical techniques to ascertain them. Additionally, the matrix itself heavily influences the selection of the right analytical method, particularly the extraction protocol. An extensive variety of plant matrices, including medicinal plants, are used to extract bioactive chemicals [7, 8], fruits and vegetables [9], cereals [10, 11], nuts [12], edible flowers [13], etc. Further, the selection of plants and compounds from among many potentially valuable natural bioactive compounds for commercialization-oriented research is a very laborious process that requires considerable knowledge and careful examination of numerous market-related issues, such as the thorough extraction and product development processes [14].

Although the classical applications of medicinal plants are well known, it is hoped that thorough scientific research into the discovery of bioactive plant components and their properties will lead to novel therapeutic treatments as well as food and cosmetic products that are derived from nature. The method devised for the identification, separation, and quantification these compounds at varied concentration levels are critical steps in any analytical approach. Further, the development of effective and economically productive technologies for identifying bioactive molecules remains a challenging task. Therefore, a thorough validation and characterization of these bioactive molecules is advantageous in revealing new sources of therapeutic agents and in determining the true significance of traditional medicines [15]. Over the past two decades, measurement techniques have grown at an unprecedented rate. Analysis of bioactive compounds is now easier than before because of incredible advancements in sophisticated analytical methods like spectroscopy, chromatography, and microscopy along with other micro-devices and sensors. General approaches in identification and characterization of bioactive molecules are depicted in Figure 1. This chapter is therefore would be of significant value for the researchers in order to create acceptable methods and techniques for chemical extraction and purification as well as to increase their yield for potential applications. The screening methods are thoroughly described, their approach is reviewed, and the existing limitations of the various methods and a look ahead to potential future advancements in characterization techniques are discussed. A range of advanced technologies along with the present methods for extracting and characterizing the bioactive molecules from certain commercial medicinal plants are also included in this chapter. The use of high resolution analytical techniques is taken into account for the characterization, quantification, and identification of bioactive compounds.

This chapter aims at the current trends and best methods for separation, identification, characterization, and quantification of bioactive molecules using various analytical approaches with specific emphasis on advanced chromatographic, spectroscopic, and spectrometric methods. The chapter highlights the analytical techniques, which includes the identification and characterization of bioactive molecules available in plants through various chromatographic techniques such as TLC (Thin Layer Chromatography), HPTLC (High Performance Thin Layer Chromatography), HPLC (High Performance Liquid Chromatography), GC (Gas Chromatography), multidimensional chromatography and spectroscopic techniques such as UV-Visible, Fourier Transform Infra-Red spectroscopy (FTIR), Nuclear Magnetic Resonance (NMR), and Mass Spectrometry (MS) along with the hyphenated techniques[16-20].

Figure 1. General approaches in identification and characterization of bioactive molecules.

8.2 Different chromatographic methods

Chromatography is known as a separation method that separate the molecules based on their size, shape, and charge by moving a mixture containing mobile phase in contact with a selectively absorbent stationary phase [21]. It is also essential as an analytical approach for quality assurance and standardized bioactive molecule production. Current applications for the separation and identification of bioactive molecules use chromatographic techniques, which exhibit great sensitivity, selectivity, and adaptability [22-24]. The chromatographic techniques have been classified in various categories based on the physical state of mobile phases (Liquid/Gas), stationary phase (Paper & Thin Layer/Column), Separation mechanism (Affinity/Partition/Size exclusion or Gel filtration /Ion exchange/) etc. While chromatographic methods can be used for both the qualitative and quantitative studies, TLC provides a preliminary evaluation of the qualitative analysis of complex mixture.

Thin layer chromatography (TLC) and preparative TLC:

TLC, an adsorption chromatography separates the compounds based on the interaction of a thin layer adsorbent adhered to the surface [25, 26]. The silicon atoms present in the silica gel have binding property through hydrogen bonding via hydroxyl groups and considered to be responsible for the separation processes. The mixture's components get separated according to the variation in the molecular functional groups' binding efficiencies which cause the mixture to attach to the silica gel. Higher polarity compounds are easily adsorbed by the silica gel, but lower polarity compounds are travel with mobile phase and appear as spot on the plate. A modification of this technique results in preparative thin layer chromatography (prep-TLC). This technique separate components in higher amount by loading more amount of sample on thick layer (0.5-5 mm) of the stationary phase.

High performance thin layer chromatography (HPTLC):

HPTLC, a planar chromatography technique separates the compounds through high performance layers along with data acquisition and detection. Pre-coated sorbent plates of 5-7 microns particle size and 150–200 thickness make up these high-performance layers. The separation efficiency increases with reduction in particle size and thickness of layer. When using HPTLC, the separated samples obtained through chromatography are visually viewed as a chromatogram. Pore and particle size of sorbents are the primary distinction between TLC and HPTLC.

Liquid chromatography (LC) and Preparative LC:

For complex mixtures separation, liquid chromatography is a proven, dependable method which frequently utilized in quality control and quality analysis. In addition to offering high resolution similar to LC, preparative liquid chromatography (PLC) also significantly increases the sample loading volume. High productivities can be achieved by severely loading columns, which is frequently done by testing solubility limits, injecting big sample volumes, or injecting large sample concentrations. As a result, it is possible to obtain good quantity of bioactive molecules from natural sources.

Flash Chromatography:

This technique for the separation and purification of chemicals is also referred to as medium pressure liquid chromatography. Mixture of bioactive molecules in large sample volumes can be separated with a controlled moderate pressure, producing highly pure molecules. The recent development of fully automated fraction collectors and real time detection has enhanced the effectiveness of separation and isolation along with purification and identification of molecules in a crude extract [27].

High performance liquid chromatography (HPLC):

By using HPLC, compounds are separated depending on their interaction with the mobile phase's solvent and the particles of a tightly packed column. In HPLC, combination of a polar liquid phase and a non-polar solid phase are both employed. High pressure must be used to elute the analyte through a column, after which it passes via a diode array detector (DAD). A DAD evaluates the analytes' absorption spectra to assist in identifying them. HPLC is helpful and works well in conjunction with gas chromatography to detect bioactive molecules for substances which do not vaporize or break down at high temperatures, [28]. UHPLC uses further high pressures which results in lesser run time and solvent consumption with better analytical separation [29].

Preparative High-Performance Liquid Chromatography (Prep-HPLC):

Prep-HPLC is used on industrial scale with kilogram quantities for separartion and purification purpose. Large columns which run at higher flow of the eluting solvents are used for this purpose. In normal HPLC, sample moves to trash after the evaluation but in Prep-HPLC, sample leaves detector and enters the fraction collector. Prep-HPLC offers an advantage over traditional HPLC because it uses more pressure and increasing throughput while ensuring adequate separation power. In comparison to previous purification methods, fully automated prep-HPLC is currently able to quickly and easily purify a significant number of important bioactive molecules. The isolation and purification process are made much more effective using an UV-Visible detector with prep-HPLC [30].

Reverse-Phase High-Performance Liquid Chromatography:

This often referred as hydrophobic chromatography because it employs a stationary phase which is hydrophobic in nature. This is distinct from normal phase chromatography that uses silica/alumina resins which are hydrophilic in nature. In reverse-phase HPLC, the hydrophobic stationary phase is made up of covalently linked alkyl groups that attach to the less polar (hydrophobic) chemicals and transported by mobile phase. Reversed-phase chromatography offers the extra benefit of separating charged molecules utilizing ion interactions in modern methods.

Gas chromatography (GC):

Gas Chromatography is utilized for the isolation and identification of complex mixtures of volatile compounds [31]. This technique includes sample vaporization and injection in a column's head that comprises of liquid stationary phase. The sample is carried by the flow of an inert, gaseous mobile phase. For example, molecules that completely disperse into the gas phase will travel at the similar speed of the gas, but molecules that completely disperse into liquid stationary phase remains behind. Species that split their distribution between the two stages will migrate at a moderate pace [32]. To distinguish between chemicals that act similarly throughout the GC process, different temperature programs may be employed to improve the analysis [15]. Due to their great sensitivity, selectivity, and versatility, mass spectrometers (MS) are one of the detectors employed most frequently today for the detection of bioactive molecules. However, due to various tactical advantages, such as linearity for broad concentration ranges, reduced prices, and low maintenance needs compared with MS, FID, ECD detectors are also frequently employed in gas chromatography. The most popular method for identifying and measuring phytochemicals is GC-MS, which is both a strong instrument and a reliable methodology. The interpretation of the spectrum and comparison with National Institute of Standards and Technology (NIST) allowed identification of bioactive species [33]. In a recent work, Mani Ganesh and Murugan Mohankumar used GC-MS to identify the bioactive substances in ethanol extracts of S. cordata whole plant [34]. Amudha et al. have also identified potential phytocompounds in ethyl acetate extract of E. acoroides using GC-MS [35].

Size-exclusion chromatography (SEC):

In this technique, molecule is separated based on its size, that further determines its molecular weight, in a process known as molecular sieve chromatography. Large molecules having complicated structures are best separated with this approach. The columns used for this process are tiny, sieve-like reticular beads made by dextran polymers, agarose (Sepharose), or polyacrylamide (Sephacryl). According to their size, the components in the extracts are caught in the beads, making it easier to estimate the sizes of macromolecules and achieve a fair molar mass distribution [36]. When an aqueous solution is used, it is called gel-filtration chromatography, and when an organic solvent is used, it is referred to as gel permeation chromatography.

Counter-current chromatography (CCC):

Traditional chromatographic techniques with lower pressure are time/solvent intensive and need numerous chromatographic steps. In some cases, CCC might be a perfect replacement for these traditional purifying techniques.  CCC has some advantages over conventional liquid-solid separation techniques, including high injection sample recovery, minimal peak tailing, low sample denaturation risk, compatibility with particulate samples, and ease of switching solvents for fresh samples. CCC is divided into hydrophobic and hydrophilic equilibrium systems depending on the various fluid mechanics concepts that apply to them. Pauli, Pro, & Friesen (2008) have written a thorough review that provides explicit details of such systems along with their key features [37]. By using different gradient elution processes, this method can be used to improve the removal of contaminants from both the raw extract/finished product and pure molecule can be isolated from raw extract in single step without the need for sample pre-treatment [38, 39].

Bio-affinity chromatography:

Bioactive molecules from natural products have already been successfully screened using the significant and well-liked technology of bio-affinity chromatography, which depends on interaction between bioactive molecules and immobilized targets [40]. Targets such as cell membrane, protein, liposome etc. are utilized as a stationary phase along with a carrier to keep the bioactive molecules which precisely attach to the target. Natural products extract is first injected in affinity column filled by carriers that are masked with the targets. An elution buffer is subsequently used to separate the bound bioactive molecules from the targets, which are then subjected to detector analysis. Cellular membrane affinity chromatography (CMAC) [41], cellular membrane chromatography (CMC) [42-44], immobilized liposome chromatography (ILC) [45], and biological molecule chromatography [46] are the type of bio-affinity chromatography that have received the most attention to date.

Modern pharmacological research has shown that the first step in predicting a drug's behavior in an organism is its capacity to connect to the receptors which is crucial in their action [47]. The concept of CMC, first introduced by He et al. gained popularity as standard cell membrane dependent model for bioactive molecule’s screening [48, 49]. Although this CMC technique is quicker and easier than traditional screening and analysis, its implementation is restricted by limited column and peak efficacy. These drawbacks have been significantly minimized by CMC and multidimensional chromatography (2D CMC-HPLC) which will be discussed in the following section of multidimensional and hyphenated chromatography techniques.

Multidimensional liquid chromatography (MD-LC):

Because of their unquestionably greater separation ability compared to the traditional 1D chromatographic method in deciphering complicated samples, multidimensional chromatography techniques continue to occupy a growing niche in the field of separation science. Even with careful method optimization, a single chromatographic process may fall short of providing a suitable level of resolving power when dealing with extremely complex samples, necessitating the use of more potent analytical techniques. The evaluation of complicated samples using MD-LC has become a compelling option. Natural products are not an exception, and due to their selectivity and sensitivity, multidimensional approaches can be used to analyse bioactive compounds as well as identify minor components. To greatly improve the isolation potential of related 1D-LC methods, MD-LC enables the combining of two or more separation processes.  General aspects such as basic principles, models, and utility of the various MD-LC approaches in natural product investigation have already been covered in several fascinating reviews [50-58]. In this section an overview of comprehensive MD-LC methods employed in offline/online mode is discussed that is used as effective tool in investigation of the natural products.

Offline L×C:

When performing off-line LC×LC, the fractions of particular interest eluted from the initial 1D separation are collected manually, subjected to evaporation, and subsequently injected into the 2D. Consequently, this approach yields a final outcome comprising multiple two-dimensional separations, corresponding in number to the fractions collected from the one-dimensional separation. The primary advantages include the ease of execution, without the necessity of alternate valves. This compatibility allows for the integration of a wide range of LC-modes.

Online L×C:

The interfaces used in online L×C can automatically transfer 1D fraction into 2D. In contrast to the offline mode, the online approach offers several notable advantages: it can be easily automated, enhancing sample throughput, while reducing the risk of sample contamination, and it results in a shorter overall run time. Furthermore, it provides greater reproducibility, and there is a significant potential for discovering "unknowns" due to the emergence of distinct group type patterns on the 2D plane.

Hyphenated (Chromatographic-Spectroscopic/ Spectrometric) techniques:

The analytical methods in which a chromatographic technique and spectroscopic or spectrometric technique are coupled to analyse a variety of samples are known as "hyphenated techniques". These are based on the target's initial extraction and/or separation using LC, HPLC, GC, etc., and their subsequent characterisation using spectroscopic techniques like FTIR, MS, NMR, etc. All efforts made over the last few decades by scientists and engineers resulted in the creation of high-tech hyphenated equipment such as LC-MS, LC-NMR, HPLC-MS, and GC-MS. As shown in Figure 2, GC-MS and LC-MS (in hydrophilic interaction/HILIC and C18 mode) are widely used in the characterization of a range of bioactive molecules. Table 1 provides a comprehensive overview of the strengths and weaknesses associated with both of these methodologies.

Figure 2. Selection of the characterization methods for bioactive molecules based on their nature and polarity.

Table 1 Advantages and limitations of GC-MS and LC-MS methods.

| Techniques | Advantages | Disadvantages |
| --- | --- | --- |
| GC-MS | Volatile components and low molecular mass hydrophobic compounds are easily analysed. | Derivatisation is required for non-volatile and thermally unstable compounds which sometime mask the results. |
| LC-MS | Suitable for characterization of relatively polar as well as thermally unstable compounds without any derivatization. By using different ionisation approaches, full range of molecular weights (low, intermediate, and high) can be detected. | Poorly reproducible fragmentation patterns and formation of adducts are the two major problems. |

Some more advanced techniques have been introduced in the recent years by coupling between chromatography and spectroscopy such as LC-MS/MS, LC-NMR-MS, GC-MS/MS etc. In this section some recent studies in the field of bioactive molecules characterization using hyphenated techniques have been discussed. The development of increasingly advanced high resolution analytical methods with enhanced sensitivity, effectiveness, and selectivity is of paramount importance. The use of LC in conjunction with various detectors, such as the photodiode array (PDA), fluorescence detector (FD), and mass spectrometry (MS), enables the structural elucidation and identification of bioactive compounds at a comprehensive level. PDA is frequently used because of its dependability and affordability, and it can be used to identify wide range of secondary metabolites. Overall, electrospray ionization-based MS detectors deliver the best performance. When paired with Mass Spectrometry (MS), chromatographic techniques have shown promise as separation and characterisation of bioactive compounds simultaneously [59, 60]. These platforms are highly selective, sensitive, quick, and high-resolution. Certain ambient MS techniques' direct analysis capabilities have proven to be particularly beneficial for the analysis, identification, and characterisation [61]. An integrated identification method incorporates various elements such as HRMS, ion source and MS/MS fragmentation, determination of molecular formulas, and data base searches. Advanced hyphenated techniques, specifically the coupling of liquid/gas chromatography (GC) with HRMS, are potent tools to tackle the intricate analysis of botanicals [62, 63]. Silylation is a derivatization method that makes the derivatives more flammable and enabling them direct injection into the GC system. Silyl group insertion can also result in a more advantageous ion fragmentation pattern that is employed for structural analysis [64]. The most common analyzers are orbitrap based hybrid mass spectrometry (LTQ orbitrap) and quadrupole time of flight (QTOF). Due of its capability to do multiple reactions monitoring (MRM), QqQ is frequently utilised for quantitative reasons. The best resolution for identifying bioactive compounds is provided by Q-TOFMS, which enables one to carry out MS/MS investigations with enhanced structural details and selectivity. Using HPLC-ESI-QTOF/MS, phenolic compounds from several matrices have been successfully identified [65-69]. Despite the widespread use of these methods, MS-based procedures produce enormous, complex datasets with hundreds of MS characteristics, leads to difficulties in post-acquisition data processing and higher time consumption [70]. To correctly recognize the possible bioactive components, raw MS data must be subjected to an integrated identification and elucidation technique. The mass-defect filtering, fragment-ion filtering, and neutral-loss filtering are a few recent research that have suggested useful methods to simplify the post-acquisition data processing [71-73]. Recently, LC-MS is used to characterise a variety of natural products, in the investigation of various plants and derived products [74-76]. The primary bioactive constituents in these extracts, as established by the most comprehensive published findings on characterizing M. oleifera leaf extract, identified a total of 59 chemical components [75]. Additionally, depending on the settings utilised, LC-MS is a promising alternate for recognizing a variety of bioactive chemicals as it may be able to distinguish between isomeric molecules [77, 78]. Using HPLC in conjunction with electrospray ionisation quadrupole-time of flight mass spectrometry (HPLC-ESI-QTOF-MS), chemical molecules can be identified by analysing MS spectra, and comparison with mass spectra database and scientific literature. These findings support previously reported qualitative findings indicating this method can be used to isolate at least 50 bioactive components from M. oleifera leaf optimised extracts [79]. Additionally, M. oleifera and M. ovalifolia's flavonoid contents were compared using ultra-HPLC in conjunction with QTOF-MS [76]. These results are supporting the similar recent study carried out by Castro-López and co-workers on M. oleifera leaf extracts [80]. They used UPLC-ESI-QTOF-MS2 to identify these bioactive compounds.  In their investigation of the combined effects of pulsed electric field (PEF) and ultrasound (US), Manzoor and co-workers observed the stability of volatile components to assess the physico-chemical properties of bioactive substances along with structural assignments of almond extract using FT-IR [81]. By using cutting-edge HPLC and GC techniques, Pellati F et al. reported multi component study of bioactive chemicals found in C. sativa (hemp) of several varieties [82]. Recent research has employed 2D-CMC-HPLC methods to screen and assess bioactive chemicals. These techniques use CMC-UV to retain possible active components in one dimension, and examined using HPLC or GC-MS in the second dimension, either offline or online. This is successfully utilized to screen and examine bioactive molecules, but it has two significant drawbacks: the molecules with little or no absorption of UV may go unnoticed by UV detectors and second dimension takes a lot of time to analyse. Chen et al. created a 2D HepG2/CMC/monolithic column/TOF/MS system to screen and characterize putative bioactive components faster to get over these constraints [83]. Moaddel et al. presented several significant studies on CMAC to screen bioactive chemicals from natural product extracts in recent years [84-86]. A recent study used HPTLC-UV/Vis/FLD-bioassay to build a simplified quantitative bio profiling investigation of 17 widely available ginger varieties. HPTLC-ESI-HRMS was also used to find and characterise multipotent chemicals. Such chromatographic techniques were used to analyse chemical marker components along with the activity of ginger extracts [87]. Recent studies used reversed-phase HPLC in conjunction with ESI-TOF-MS to analyse the bioactive molecules present in citrus by-products. Six new metabolites are reported for the first time in citrus by-products out of the forty or more that could be determined using the method [88]. For the purpose of screening chemical components in natural products, a number of online SEC-HPLC-DAD and SEC-HPLC-MS systems are recently developed [89-91]. Using UPLC-ESI-q-TOF-MS/MS technique a thorough characterization of P. peruviana extract has been developed and a total of 59 phytochemicals were provisionally discovered. Metabolite profiling of brown macro-algae has recently established through the application of UPLC-MS [92-94].

8.3 Different spectroscopic methods

Although chromatographic techniques are widely considered the benchmark for analyzing bioactive molecules, spectroscopic methods hold equal significance since they can effectively offer insights into all species within a sample in a single experiment. Electromagnetic radiation used in spectroscopy travels via organic molecules which partially absorb the incident light and partially transmit the remainder. The measurement involves assessing the electromagnetic energy that has been either absorbed or transmitted, and the subsequent spectrum is utilized for analysis. Particular bonds within the molecule are responsible for absorbing specific electromagnetic radiation. Consequently, spectra are generated from various electromagnetic regions, including UV-Visible, infrared, radiofrequency etc. Several different spectroscopic methods, such as UV-Vis., FTIR, NMR and mass are employed to ascertain the structural composition of bioactive molecules that have been isolated from plants. Atomic adsorption spectroscopy (AAS) and inductively coupled plasma mass spectrometry (ICP-MS) or inductively coupled plasma optical emission spectroscopy (ICP-OES) are used to analyse or determine the heavy metals and other elements present. In the following section the spectroscopic techniques like UV-Vis., FTIR, NMR and mass that are used in the structure determination of the bioactive molecules will be discussed with specific emphasis on the recent advancements in instrumentation and methodologies.

UV-Visible Spectroscopy

UV-visible spectroscopy is versatile in its ability to perform both qualitative and quantitative analysis and to identify specific classes of substances. This technique is especially advantageous for quantitative studies due to the strong chromophoric nature of aromatic compounds in the UV range. Additionally, UV-visible spectroscopy proves valuable in identifying natural substances, as it can detect saturation in molecules containing aromatic moieties and other chromophores that absorb in the UV region. The UV-Visible spectrum allows for the identification of phenolic substances such as sugars, pigments, flavonoids, coumarins, anthocyanins, tannins etc. [95]. This approach is not only cost-effective but also less time-consuming compared to alternative methods [96].

Fourier-transform infrared spectroscopy (FTIR)

Infrared spectroscopy utilizing the Fourier transform is a significant technique in locating functional group of bioactive molecules. It provides high-resolution analytical data to identify the complicated compounds. It contributes to the recognition of molecular properties and assists in determining the structure of molecules. The type of vibrational shifts that a polar molecule experiences in response to IR radiation determines how well it absorbs that energy. Therefore, it is also named as vibrational spectroscopy. When IR radiation passes via a sample, certain frequencies get absorbed, and others will pass via sample.

The molecule being studied should exhibit polarity, characterized by a dipole moment to receive a response from IR spectroscopy. The distinctive absorption bands at specific frequencies in the infrared spectrum can be utilized to ascertain the presence of these types of bonds in an organic molecule [97]. FTIR is an efficient, cost effective, and non-destructive investigative technology which can provide fingerprints for the structural information of isolated bioactive compounds. There are several techniques available for sample preparation in FTIR analysis. The easiest approach for liquid samples involves placing a drop of the sample between two sodium chloride plates, creating a thin layer between them. Solid materials are often milled with potassium bromide (KBr), producing a thin pellet that can be analyzed. In the case of solid samples without a solvent, they can be dissolved in a suitable solvent. A few drops of the resulting solution are then applied to an attenuated total reflectance (ATR) plate, and spectra are recorded. To determine the signatures of main functional groups and bondings, the peaks are examined in relation to the spectral band locations. The acquired spectra were analyzed, and the principal bands were assigned by comparing them to the available literature.  Ismail et al. recently investigated and measured bioactive molecules present within grapes seeds using FTIR. This group discovered carboxylate group from proanthocyanidingallate and gallic acid within the water extract of the seeds [98]. The functional moieties observed in extract were qualitatively analyzed using the ATR-FTIR technique, and the samples were authenticated and distinguished by employing a multivariate statistical technique known as Principal Component Analysis (PCA) in which multi-dimensionality of a data set are reduced to 2-D or 3-D data/graph by computing the eigenvalue of the covariance matrix [99, 100].

NMR investigation of bioactive molecules

Nuclear Magnetic Resonance spectroscopy, a uniquely capable non-destructive technique provides comprehensive analysis of structure, conformational analysis dynamics, molecular interaction and three-dimensional structure of simple to complex molecular system in both solution and solid state. Moreover, continuous developments in methodology, introduction of high field super conducting magnets, FT and PFG techniques have has established it as one of the most preferred methods for structural investigation of bioactive molecules at atomic resolution [101, 102]. Due to this, NMR spectrometers are now widely employed as a primary tool in laboratories that focuses on bioactive compounds. The complex structural details of the molecules can be obtained using 2D NMR methods instead of the frequently utilized one dimensional methodology. The common NMR methods used in the characterization of bioactive molecules are shown in Table 2 [103-105].

Table 2. Common NMR methods with potential use in the characterization of bioactive molecules.

| S.N. | NMR Experiment | Application |
| --- | --- | --- |
| Solution state NMR |  |  |
| 1. | Chemical shifts | Chemical structure and stoichiometry. |
| 2. | COSY | Correlates protons that are either geminal or vicinal. |
| 3. | Magnetization transfer (NOESY/ROESY/EXSY) | Spatial interactions, structural/ conformational determination, internuclear distance measurement and chemical exchange. |
| 4. | TOCSY | Identifies proton signals within the same spin system. |
| 5. | HSQC | Determines the multiplicity of carbon nuclei and establishes correlations between carbon signals and directly bound protons. |
| 6. | HMBC | Enables the identification of long range correlations. |
| 7. | Relaxation times T1/T2 | Molecular mobility and spatial interactions. |
| 8. | Magnetization transfer (NOESY/ROESY/EXSY) | Spatial interactions, structural/ conformational determination, internuclear distance measurement and chemical exchange. |
| 9. | DOSY | NMR signals are separated according to the diffusion coefficient of molecular system. |
| Solid state NMR |  |  |
| 10. | CP-MAS | Spectra of hetero-nuclei X that are directly bound to hydrogen atoms. |
| 11. | HETCOR | Correlate heteronuclear resonance frequencies via their one bond couplings. |
| 12. | CPXT1 | Dynamic character of the molecule. |

Compounds must first be chromatographically purified before they may be detected by NMR spectroscopy. NMR is often incorporated into the separation, isolation, and purification procedures utilizing the preparative, semi-preparative, thinlayer, and HPLC to elucidate the structural details of the compounds. By connecting the hydrogen signals in aromatic compounds with their corresponding carbon signals via 13C NMR spectroscopy, HMBC is used to provide more light on these structures. These methodologies are very crucial because the biological activity of bioactive molecules may be greatly influenced by their structural integrity. In order to explore the metabolomics profile of citrus juices, Villa-Ruano and co-workers have used the 1H NMR methods to collect spectra for 35 metabolites [106]. Cérantola and co-workers utilized HSQC and HMBC 2D NMR experiments to determine structures of several bioactive compounds in Fucus spiralis [107]. Several bioactive molecules have been quantitatively measured in brown algae Cystoseira tamariscifolia using HRMAS SS- NMR [108]. The application of SS- NMR in the investigation of brown algae is extremely beneficial as it can be analysed without the requirement of solvent extraction and eliminates the associated matrix effects. The same scientists also examined a brown macroalgae Cystoseria of the Fucaceae family, and reported the comparision between LC/ESIMS and 1H HRMAS experiments [109]. The concentration of phloroglucinol was qualitatively measured using 1H HRMAS in conjunction with 1H qNMR. Cystoseria fingerprints were discovered to be helpful in differentiating between the studied species, which were never able to be separated because of identical metabolic profiling. Cicero et al. used HRMAS NMR for quantitative investigation of main metabolites found in two hybrid varieties of citrus fruits i.e., Messina and Turkish lemon [110].  In recent years, NMR based metabolomics for the application of functional genomics and plant differentiation have also gained considerable attention. This protocol has the sample preparation, extraction, NMR analysis, and chemometric procedures steps used in plant metabolomics employing NMR spectroscopy [111-113].  Dinesh Kumar and co-workers performed thorough metabolite profiling of numerous tissues from the Crataegus rhipidophylla Gand plant, using 1D and 2D NMR methods and identified 58 compounds belonging to various classes [114]. The ability to identify metabolites via comparison of NMR spectra with standards or via employing 2D NMR to elucidate structure is main benefit of NMR metabolomic studies. This procedure is best suited for examination of primary and secondary metabolites as well as phenolic compounds, which are often plentiful in plants.

Mass spectrometry (MS):

MS is a potent method of identifying the unknown molecules, quantifying known molecules, and determining the structural and chemical characteristics of molecules. This method uses electrons or lasers to bombard organic molecules and turning them into charged ions with high energy. The internal electric and magnetic fields of the instrument segregate different-sized fragments based on their mass to charge (m/z) ratio. Energetic species pass towards detector with varying kinetic energy according to their mass. To create a mass spectrum, the relative abundance of fragmented ion is plotted against the ion's m/z ratio. Mass spectrometry allows for the precise calculation of relative molecular mass and the determination of an accurate molecular formula by analyzing molecular fragmentation patterns. This technique is primarily utilized for the structural analysis of organic compounds and for monitoring the presence of known compounds in complex mixtures with a high level of specificity by simultaneously defining the molecular weight and diagnostic fragments of the molecules. Assignment of bioactive molecules is typically based on parent ion's size. Mass spectrometry provides a wealth of data for the structural determination when applied as tandem mass spectrometry, or MS/MS. General setup of a typical tandem mass spectrometer is depicted in Figure 3. It has frequently been employed in conjunction with different LC techniques to track the pattern of compound fragmentation as it elutes on the column [115-117].

Figure 3. General setup of a typical tandem mass spectrometer.

The HPLC and MS combination makes it easier to identify chemical components quickly and accurately even in the absence of purified standards. This method has recently been used for elucidating the structure of bioactive compounds recovered from citrus waste [59]. The type of the ionisation source is also crucial for the investigation. Enhancing the analysis's efficiency, including improved sensitivity and the ability to conduct high-throughput analysis of phytochemical compounds, depends on the selection of various ionization sources. These sources serve as interfaces between LC MS. Some of the commonly employed ionization sources for the fragmentation of complex botanicals include electron impact (EI), fast atom bombardment (FAB), matrix-assisted laser desorption/ionization (MALDI), electrospray ionization (ESI), atmospheric pressure chemical ionization (APCI), and, more recently, atmospheric pressure photoionization, as well as atmospheric pressure solids analysis probe (ASAP [118]. LC-MS is widely employed to analyse phenolic compounds using ESI as a recommended source because of its higher ionization efficiency. For assessing bioflavonoids, ESI/MS and APCI/MS offer significant advantages in terms of mass scale and sensitiveness [119]. Charge transfer through dopant compound to analyte occurs during LC/MS-APCI method, and analyte is then ionized utilizing photons through a vacuum UV lamp. Analysing non-volatile molecules with MALDI is helpful [120]. For bioactive molecules with higher degrees of polymerization including bioflavonoids, peptides, sugars, carbohydrates, glycoconjugates and proteins, it is recommended to adopt MALDITOF, an ionization technology which utilize laser light to create ions from big compounds without fragmentation [121, 122]. It has also been demonstrated that MALDI has the potential to analyse saponins that have been isolated from alfalfa [123]. This useful method can offer a direct examination of plant extracts without requiring any separation or derivatization steps [124]. Additionally, it was shown that MALDI can differentiate between different oligosaccharide isomers [125, 126]. Because of the appearance of the peaks as numerous charged species, HRMS can also quantify larger bioactive molecules with higher degrees of polymerization. The determination of phenolic compounds using HPLC coupled to UV-PDA and ESI/MS has been proven to be effective [127]. Their potential molecular structural details are further validated by using NMR spectroscopy. The primary applications of GC/MS are for volatile compounds such essential oils. Due to less volatile nature of flavonoid glycosides, this is seldom employed in flavonoid investigation. APCI has been proven to be quite helpful in the analysis of carotenoids since it offers increased sensitivity towards the charged ions. APCI's great sensitivity has led to it becoming the most popular ionization method for carotenoids.

8.4 Summary

There are numerous methods for identifying, isolating, and extracting bioactive substances from plants. There is a pressing demand for innovative, cutting-edge approaches for extraction, identification and isolation of bioactive chemicals in amounts suitable for their significant uses in a variety of fields, yet these methods are typically time-consuming and extremely expensive. A wide variety of bioactive chemical substances, either in its purest form or as homogeneous extracts, have been produced from medicinal plants. These compounds provide pharmaceutical companies with several prospects for the creation of new therapeutic leads due to their wide structural and functional diversity. The evolution of novel and more efficient investigation and separation methods is becoming extremely important as the food, pharmaceutical, and cosmetics sectors become more and more interested in naturally occurring bioactive molecules and secondary plant metabolites. The goal when searching for bioactive molecules is to identify a suitable method that can rapidly screen the source material for various bioactivities like antioxidant, antibacterial, or cytotoxic properties, while ensuring simplicity, specificity, and speed in the process. The first stage in isolating all biologically active compounds is selecting the optimum extraction technique to maximise yield while utilising the least expensive solvent volumes, time, and energy requirements. Different techniques function differently as per the nature of the target components. An overview of the characterization methods available for biologically active molecule is presented in this chapter, with a focus on contemporary chromatographic and spectroscopic methods. Each technique's advantages and disadvantages are explored. The methodologies that have been outlined here can provide valuable insight for a comprehensive investigation of plant raw components as well as the bio-actives. Natural product researchers may find the information in this chapter to be a useful resource for producing better pharmaceutical products using the most potent natural bioactive constituents.

References

- Roemer T, Xu DM, Singh SB, Parish CA, Harris G, Wang H, Davies JE, Bills GF. Confronting the challenges of natural product-based antifungal discovery. Chem Biol. 2011;18:148–64.

- Lu Q, Jiang JG. Chemical metabolism of medicinal compounds from natural botanicals. Curr Med Chem. 2012; 19:1682–705.

- Zhang YF, Xiao S, Sun LJ, Cheng YY. Rapid screening of bioactive compounds from natural products by integrating 5-channel parallel chromatography coupled with on-line mass spectrometry and micro plate based assays. Anal Chim Acta. 2013;777:49–56

- Butler MS, Fontaine F, Cooper MA. Natural product libraries: assembly, maintenance, and screening. Planta Med. 2014;80:1161–70.

- Bernhoft A. Bioactive compounds in plants– benefits and risks for man and animals. The Norwegian Academy of Science and Letters: Oslo, Norway. 2008:1–255.

- Bubalo CM, Vidović S, Redovniković IR, Jokić S. New perspective in extraction of plant biologically active compounds by green solvents. Food Bioprod Process. 2018;109:52-73.

- Patra JK, Das G, Lee S, Kang SS, Shin HS. Selected commercial plants: A review of extraction and isolation of bioactive compounds and their pharmacological market value. Trends Food Sci Technol. 2018;82:89–109.

- Ivanović M, Razboršek MI, Kolar M. Innovative Extraction Techniques Using Deep Eutectic Solvents and Analytical Methods for the Isolation and Characterization of Natural Bioactive Compounds from Plant Material. Plants (Basel). 2020;9(11):1428-57.

- Barba FJ, Mariutti LRB, Bragagnolo N, Mercadante AZ, Barbosa-Cánovas GV, Orlien V. Bioaccessibility of bioactive compounds from fruits and vegetables after thermal and nonthermal processing. Trends Food Sci Technol 2017;67:195–206.

- Mansur AR, Song NE, Jang HW, Lim TG, Yoo M, Nam TG. Optimizing the ultrasound-assisted deep eutectic solvent extraction of flavonoids in common buckwheat sprouts. Food Chem 2019;293:438–45.

- Shang X, Dou Y, Zhang Y, Tan JN, Liu X, Zhang Z. Tailor-made natural deep eutectic solvents for green extraction of isoflavones from chickpea (Cicer arietinum L.) sprouts. Ind. Crops Prod. 2019;140:111724.

- Gorji N, Moeini R, Memariani Z. Almond, hazelnut and walnut, three nuts for neuroprotection in Alzheimer’s disease: A neuropharmacological review of their bioactive constituents. Pharmacol Res. 2018;129:115–27.

- Skrajda-Brdak M, Da˛browski G, Konopka I. Edible flowers, a source of valuable phytonutrients and their pro-healthy effects – A review. Trends Food Sci Technol. 2020;103:179–99.

- Belwal T, Ezzat SM, Rastrelli L, Bhatt ID, Daglia M, Baldi A, Devkota HP, Orhan IE, Patra JK, Das G. A Critical Analysis of Extraction Techniques Used for Botanicals: Trends, Priorities, Industrial Uses and Optimization Strategies. TrAC, Trends Anal Chem. 2018;100,82-102.

- Al-Rubaye AF, Hameed IH, Kadhim MJ. A Review: Uses of Gas Chromatography-Mass Spectrometry (GC-MS) Technique for Analysis of Bioactive Natural Compounds of Some Plants. International Journal of Toxicological and Pharmacological Research. 2017; 9(1):81-5.

- Oleszek W, Marston A. Saponins in food and medicinal plants: Kluwer academic publishers New York. 2000.

- Schefer AB, Braumann U, Tseng LH, Spraul M, Soares MG, Fernandes JB, da Silva MFGF, Vieira PC, Ferreira AG. Application of high-performance liquid chromatography– nuclear magnetic resonance coupling to the identification of limonoids from mahogany tree (Switenia macrophylla, Meliaceae) by stopped-flow 1D and 2D NMR spectroscopy. J Chromatogr A. 2006;1128:152-63.

- Doughari JH. Phytochemicals: Extraction methods, basic structures and mode of action as potential chemotherapeutic agents. In Phytochemicals-A global perspective of their role in nutrition and health: InTech. 2012.

- Chikezie PC, Ibegbulem CO, Mbagwu FN. Bioactive principles from medicinal plants. Res J Phytochem. 2015;9:88-115.

- Patra JK, Das G, Lee S, Kang SS, Shin HS. Selected commercial plants: A review of extraction and isolation of bioactive compounds and their pharmacological market value. Trends Food Sci Technol. 2018;82:89-109.

- Heftmann F. Chromatography: Fundamentals and Application of Chromatographic and Electrophoretic Techniques. 5th edn., Elsevier, Amsterdam, The Netherlands. 1992.

- Oleszek W, Bialy Z. Chromatographic determination of plant saponins--an update (2002-2005). J Chromatogr A. 2006;1112(1-2):78-91.

- Ignat I, Volf I, Popa VI. A critical review of methods for characterisation of polyphenolic compounds in fruits and vegetables. Food Chem. 2011;126(4):1821-35.

- Al-Suod H, Ligor M, Rațiu IA, Rafińska K, Górecki R, Buszewski B. A window on cyclitols: Characterization and analytics of inositols. Phytochem. Lett. 2017, 20, 507-519.

- Bolliger HR, Brenner M, Gänshirt H, Mangold HK, Seiler H, Stahl E, Waldi D. Thin-Layer Chromatography: A Laboratory Handbook, Springer Berlin, Heidelberg. 1965.

- Hahn-Deinstrop E. Applied Thin Layer Chromatography: Best practice and avoidance of Mistakes. Wiley-VCH, Weinheim, Germany. 2000.

- Uckoo RM, Jayaprakasha GK, Patil BS. Chromatographic techniques for the separation of polymethoxyflavones from citrus. In Emerging Trends in Dietary Components for Preventing and Combating Disease; ACS Symposium Series; American Chemical Society: Washington, DC, USA, 2012:3–15.

- Katz ED. High Performance Liquid Chromatography:  Principle and Methods in Biotechnology (Separation science Series). John wiley & sons, New Jersey, USA. 1995.

- Ares AM, Valverde S, Bernal JL, Nozal MJ, Bernal J. Extraction and determination of bioactive compounds from bee pollen. J Pharm Biomed Anal. 2018;147:110-124.

- Huber U, Majors RE. Principles in Preparative HPLC. Available online: https://www.agilent.com/cs/library/ primers/Public/5989-6639EN.pdf

- Littlewood AB. Gas Chromatography Principle, Techniques and Applications. Academic Press, London, U.K. 1962.

- Burchfield AP, Storrs EE. Biochemical Applications of Gas chromatography. Academic press, New York., USA. 1962.

- Hites AR. Gas Chromatography Mass Spectroscopy: Handbook of Instrumental Techniques for Analytical Chemistry. New Jersey: Prentice Hall PTR; 1997:609-11.

- Ganesh M, Mohankumar M. Extraction and identification of bioactive components in Sida cordata (Burm.f.) using gas chromatography-mass spectrometry. J Food Sci Technol. 2017;54(10):3082-91.

- Amudha P, Jayalakshmi M, Pushpabharathi N, Vanitha V. Identification of bioactive components in enhalus acoroides seagrass extract by gas chromatography-mass spectrometry. Asian J Pharm Clin Res. 2018; 11(10):313-7.

- Zhang W, Gao L, Shi X, Zhang Q. Determination of the molecular mass of modified citrus pectin using high performance size exclusion chromatography. Chin. J. Chromatogr (Se Pu). 2007;25(5)711–714.

- Pauli GF, Pro SM, Friesen JB. Counter current separation of natural products. J Nat Prod. 2008;71:1489-508.

- Duo-Long DI, Zheng YY, Xiao-Fen C, Huang XY, Shi-Lan, F. Advances in application of high-speed counter current chromatography in separation and purification of flavonoids. Chin J Anal Chem. 2011;39:269–75.

- Wen P, Hu TG, Linhardt RJ, Liao ST, Wu H, Zou YX. Mulberry: A review of bioactive compounds and advanced processing technology. Trends Food Sci Technol. 2019;83:138-58.

- Moraes de MC, Cardoso CL, Seidl C, Moaddel R, Cass QB. Targeting anti-cancer active compounds: affinity-based chromatographic assays, Curr Pharm Des. 2016;22:5976–87.

- Rodriguez EL, Poddar S, Iftekhar S, Suh K, Woolfork AG, Ovbude S, Pekarek A, Walters M, Lott S, Hage DS. Affinity chromatography: A review of trends and developments over the past 50 years, J Chromatogr B. 2020;1157:122332.

- Hou XF, Wang SC, Zhang T, Zhang J, Zhang YM, Lu W, He HZ, He LC. Recent advances in cell membrane chromatography for traditional Chinese medicines analysis. J Pha Biomed Ana. 2014;101:141–50.

- He JC, Huang MY, Wang DM, Zhang ZM, Li GK. Magnetic separation techniques in sample preparation for biological analysis: a review. J Pharm Biomed Anal. 2014;101:84–101.

- Sun M, Ma WN, Guo Y, Hu ZG, He LC. Simultaneous screening of four epidermal growth factor receptor antagonists from curcuma longa via cell membrane chromatography online coupled with HPLC-MS. J Sep Sci. 2013;36:2096–103.

- Wang SW, Wang C, Zhao X, Mao SL, Wu YT, Fan GR. Comprehensive two-dimensional high performance liquid chromatography system with immobilized liposome chromatography column and monolithic column for separation of the traditional Chinese medicine Schisandra chinensis. Anal Chim Acta. 2012;713:121–9.

- Peng MJ, Shi SY, Chen L, Zhang SH, Cai P, Cen XQ. Online coupling solid-phase ligand-fishing with high-performance liquid chromatography-diode array detector-tandem mass spectrometry for rapid screening and identification of xanthine oxidase inhibitors in natural products. Anal Bioanal Chem. 2016;8:6693–701.

- Zheng ZG, Duan TT, He B, Tang D, Jia XB, Wang RS, Zhu JX, Xu YH, Zhu Q, Feng L. Macrophage biospecific extraction and HPLC-ESI-MSn analysis for screening immunological active components in Smilacis Glabrae Rhizoma. J Pharm Biomed Anal. 2013;77:44–48.

- He L, Wang SC, Geng XD. Coating and fusing cell membranes onto a silica surface and their chromatographic characteristics. Chromatographia. 2001;54:71–76.

- He LC, Yang G, Geng X. Enzymatic activity and chromatographic characteristics of the cell membrane immobilized on silica surface. Sci Bull. 1999;44:826–83.

- Stoll DR, Li X, Wang X, Carr PW, Porter SEG, Rutan SC. Fast, comprehensive two-dimensional liquid chromatography. J Chromatogr A. 2007;168:3-43.

- Dugo P, Cacciola F, Kumm T, Dugo G, Mondello L. Comprehensive multidimensional  liquid chromatography: theory and applications. J Chromatogr A. 2008;1184:353-68.

- Guiochon G, Marchetti N, Mriziq K, Shalliker RA. Implementations of two dimensional liquid chromatography. J Chromatogr A. 2008;1189:109-68.

- Herrero M, Ibáñez E, Cifuentes A, Bernal J. Multidimensional chromatography in food analysis. J Chromatogr A. 2009;1216:7110-29.

- Jandera P. Programmed elution in comprehensive two- dimensional liquid chromatography. J Chromatogr A. 2012;1255:112-29.

- Cacciola F, Beccaria M, Donato P, Mondello L, Dugo P. Combining orthogonal  comprehensive two-dimensional liquid chromatography methods in food analysis. LC GC Eur. 2014;27:570-7.

- Li D, Jakob C, Schmitz O. Practical considerations in comprehensive two-dimensional liquid chromatography systems (LCxLC) with reversed-phases in both dimensions. Anal  Bioanal Chem. 2015;407:153-67.

- Česla P, Křenková J. Fraction transfer process in on-line comprehensive two dimensional liquid-phase separations. J Sep Sci. 2017;40:109-23.

- Cacciola F, Dugo P, Mondello L. Multidimensional liquid chromatography in food analysis. TrAC Trends Anal Chem. 2017;96:116-123.

- Mahato N, Sinha M, Sharma K, Koteswararao R, Cho MH. Modern Extraction and Purification Techniques for Obtaining High Purity Food-Grade Bioactive Compounds and Value-Added Co-Products from Citrus Wastes. Foods. 2019;8(11):523.

- Fu Y, Luo J, Qin J, Yang M. Screening techniques for the identification of bioactive compounds in natural products. J Pharm Biomed Anal. 2019;168:189–200.

- Brown HM, McDaniel TJ, Fedick PW, Mulligan CC. The current role of mass spectrometry in forensics and future prospects. Anal Methods. 2020;12:3974–97.

- Zhang A, Sun H, Yan G, Wang X. Recent developments and emerging trends of mass spectrometry for herbal ingredients analysis. TrAC Trends Anal Chem. 2017;94:70–6.

- Zhu MZ, Chen GL, Wu JL, Li N, Liu ZH, Guo MQ. Recent development in mass spectrometry and its hyphenated techniques for the analysis of medicinal plants. Phytochem Anal. 2018;29:365–74.

- Schummer C, Delhomme O, Appenzeller BMR, Wennig R, Millet M. Talanta. 2009;77:1473–82.

- Rodríguez-Pérez C, Mendiola JA, Quirantes-Piné R, Ibáñez E, Segura-Carretero A. Green downstream processing using supercritical carbon dioxide, CO2-expanded ethanol, and pressurized hot water extractions for recovering bioactive compounds from Moringa oleifera leaves. J Supercrit Fluids. 2016;116:90–100.

- Rodríguez-Pérez C, Quirantes-Piné R, Contreras Mdel M, Uberos J, Fernández-Gutiérrez A, Segura-Carretero A. Assessment of the stability of proanthocyanidins and other phenolic compounds in cranberry syrup after gamma-irradiation treatment and during storage. Food Chem. 2015;174:392–99.

- López-Cobo A, Alberto FG, Ana María GC, Antonio SC, Federica P, María Fiorenza C. HPLC-DAD-ESI-QTOF-MS and HPLC-FLD-MS as valuable tools for the determination of phenolic and other polar compounds in the edible part and by-products of avocado. LWT. 2016;73:505–13.

- Câmara JS, Albuquerque BR, Aguiar J, Corrêa RCG, Gonçalves JL, Granato D, Pereira JAM, Barros L, Ferreira ICFR. Food Bioactive Compounds and Emerging Techniques for Their Extraction: Polyphenols as a Case Study. Foods. 2021;10:37.

- Figueroa JG, Borrás-Linares I, Lozano-Sánchez J, Segura-Carretero A. Comprehensive identification of bioactive compounds of avocado peel by liquid chromatography coupled to ultra-high-definition accurate-mass Q-TOF. Food Chem. 2018;245:707-16.

- Ren D, Ran L, Yang C, Xu M, Yi L. Integrated strategy for identifying minor components in complex samples combining mass defect, diagnostic ions and neutral loss information based on ultra-performance liquid chromatography-high resolution mass spectrometry platform: Folium Artemisiae Argy. J Chromatogr A. 2018;1550:35-44.

- Zhao W, Shang Z, Li Q, Huang M, He W, Wang Z, Zhang J. Rapid Screening and Identification of Daidzein Metabolites in Rats Based on UHPLC-LTQ-Orbitrap Mass Spectrometry Coupled with Data-Mining Technologies. Molecules. 2018;23(1):151

- Shang Z, Cai W, Cao Y, Wang F, Wang Z, Lu J, Zhang J. An integrated strategy for rapid discovery and identification of the sequential piperine metabolites in rats using ultra high-performance liquid chromatography/high resolution mass spectrometery. J Pharm Biomed Anal. 2017;146:387–401.

- Ballesteros-Vivas D, Alvarez-Rivera G, Ib´a˜nez E, Parada- Alfonso F, Cifuentes A, A multi-analytical platform based on pressurized-liquid extraction, in vitro assays and liquid chromatography/gas chromatography coupled to high resolution mass spectrometry for food by-products valorisation. Part 2: Characterization of bioactive compounds from goldenberry (Physalis peruviana L.) calyx extracts using hyphenated techniques. J Chromatogr A. 2019;1584:144-54.

- Amaglo, NK, Bennett RN, Lo Curto RB, Rosa EAS, Lo Turco V, Giuffrida A, Lo Curto A, Crea F, Timpo GM. Profiling selected phytochemicals and nutrients in different tissues of the multipurpose tree Moringa oleifera L., grown in Ghana. Food Chem. 2010;122(1):1047–54.

- Rodríguez-Pérez C, Quirantes-Piné R, Fernández-Gutiérreza A, Segura-Carretero A. Optimization of extraction method to obtain a phenolic compounds-rich extract from Moringa oleifera Lam leaves. Ind Crops Prod. 2015; 66:246–54.

- Makita Ch, Chimuka L, Steenkamp P, Cukrowska E, Madala E. Comparative analyses of flavonoid content in Moringa oleifera and Moringa ovalifolia with the aid of UHPLC-qTOF-MS fingerprinting. S Afr J Bot. 2016;105:116–22.

- Makita C, Chimuka L, Cukrowska E, Steenkamp PA, Kandawa-Schutz M, Ndhlala A. R, Madala NE. UPLC-qTOF-MS profiling of pharmacologically important chlorogenic acids and associated glycosides in Moringa ovalifolia leaf extracts. S Afr J Bot. 2017;108:193–9.

- Saucedo-Pompa S, Torres-Castillo JA, Castro-López C, Rojas R, Sánchez-Alejo EJ, Ngangyo-Heya M, Martínez-Ávila GCG. Moringa plants: Bioactive compounds and promising applications in food products. Food Res Int. 2018;111:438-50.

- Rodríguez-Pérez C, Mendiola JA, Quirantes-Piné R, Ibáñez E, Segura-Carretero A. Green downstream processing using supercritical carbon dioxide, CO2-expanded ethanol and pressurized hot water extractions for recovering bioactive compounds from Moringa oleifera leaves. J Supercrit Fluids. 2016; 116:90–100.

- Castro-López C, Ventura-Sobrevilla JM, González-Hernández MD, Rojas R, Ascacio-Valdés JA, Aguilar CN, Martínez-Ávila GCG. Impact of extraction techniques on antioxidant capacities and phytochemical composition of polyphenol-rich extracts. Food Chem. 2017;237:1139–48.

- Manzoor MF, Zeng XA, Rahaman A, Siddeeg A, Aadil RM, Ahmed Z, Li J, Niuet D. Combined impact of pulsed electric field and ultrasound on bioactive compounds and FT-IR analysis of almond extract. J Food Sci Technol. 2019;56:2355–64.

- Pellati F, Brighenti V, Sperlea J, Marchetti L, Bertelli D, Benvenuti S. New Methods for the Comprehensive Analysis of Bioactive Compounds in Cannabis sativa L. (hemp). Molecules. 2018;23(10):2639.

- Chen XF, Cao Y, Lv DY, Zhu ZY, Zhang JP, Chai YF. Comprehensive two-dimensional HepG2/cell membrane chromatography/mono-lithic column/time-of-flight mass spectrometry system for screening anti-tumor components from herbal medicines. J Chromatogr A. 2012;1242:67–74.

- Moaddel R, Hamid R, Patel S, Bullock PL, Wainer IW. Identification of P-glycoprotein substrates using open tubular chromatography on animmobilized P-glycoprotein column: comparison of chromatographic results with Caco-2 permeability. Anal Chim Acta. 2006;578:25–30.

- Maciuk A, Moaddel R, Haginaka J, Wainer IW. Screening of tobacco smoke condensate for nicotinic acetylcholine receptor ligands using cellular membrane affinity chromatography columns and missing peak chromatography. J Pharm Biomed Anal. 2008;48:238–46.

- Ciesla L, Moaddel R. Comparison of analytical techniques for the identification of bioactive compounds from natural products. Nat Prod Rep. 2016;33:1131–45.

- Krüger S, Bergin A, Morlock GE. Effect-directed analysis of ginger (Zingiber officinale) and its food products, and quantification of bioactive compounds via high-performance thin-layer chromatography and mass spectrometry. Food Chem. 2018;243:258-68.

- Gargouri B, Ammar S, Verardo V, Besbes S, Segura-Carretero A, Bouaziz M. RP-HPLC–DAD-ESI-TOF–MS based strategy for new insights into the qualitative and quantitative phenolic profile in Tunisian industrial Citrus limon by-product and their antioxidant activity. Eur Food Res. Technol. 2017;243:1–14.

- Li DQ, Zhao J, Li SP, Zhang QW. Discovery of xanthine oxidase inhibitors from a complex mixture using an online, restricted-access material coupled with column-switching liquid chromatography with a diode-array detection system, Anal Bioanal Chem. 2014;406:1975–84.

- Annis DA, Athanasopoulos J, Curran PJ, Felsch JS, Kalghatgi K, Lee WH, Nash HM, Orimination JPA, Rosnet KE, Shipps Jr GW, Thaddupathy GRA. An affinity selection–mass spectrometry method for the identification of small molecule ligands from self-encoded combinatorial libraries: discovery of a novel antagonist of E. coli dihydrofolate reductase. Int J Mass Spectrom. 2004;238:77–83.

- Flarakos J, Morand KL, Vouros P. High-throughput solution-basedmedicinal library screening against human serum albumin. Anal Bioanal Chem. 2005;77:1345–53.

- Tierney MS, Soler‐Vila A, Rai DK, Croft AK, Brunton NP, Smyth TJ. UPLC‐MS profiling of low molecular weight phlorotannin polymers in Ascophyllum nodosum, Pelvetia canaliculata and Fucus spiralis. Metabolomics. 2014;10(3):524‐35.

- Novakova L, Matysova L, Solich P. Advantages of application of UPLC in pharmaceutical analysis. Talanta. 2006;68(3):908‐18.

- Ford L, Theodoridou K, Sheldrake GN, Walsh PJ. A critical review of analytical methods used for the chemical characterisation and quantification of phlorotannin compounds in brown seaweeds. Phytochem Anal.. 2019;30:587–99.

- Kemp W. Energy and electromagnetic spectrum. In Organic Spectroscopy. Macmillan Press: London, UK. 1991:1–7.

- Urbano M, Luque de Castro MD, Perez PM, Garcia-Olmo J, Gomez-Nieto MA. Ultraviolet–visible spectroscopy and pattern recognition methods for differentiation and classification of wines. Food Chem. 2006;97:166–75.

- Kemp W. Infrared spectroscopy. In Organic Spectroscopy. Macmillan Press Ltd. London, UK. 1991:19–56.

- Ismail EH, Khalil MMH, Al Seif FA, El-Magdoub F. Biosynthesis of gold nanoparticles using extract of grape (Vitis vinifera) leaves and seeds. Prog Nanotechnol Nanomater. 2014;3:1–12.

- Pérez-Navarro J, Da Ros A, Masuero D, Izquierdo-Cañas PM, Hermosín-Gutiérrez, Gómez-Alonso S, Mattivi F, Vrhovsek U. LC-MS/MS analysis of free fatty acid composition and other lipids in skins and seeds of Vitis vinifera grape cultivars. Food Res Int 2019;125:108556.

- Lucarini M, Durazzo A, Kiefer J, Santini A, Lombardi-Boccia G, Souto EB, Romani A, Lampe A, Ferrari Nicoli S, Gabrielli P, Bevilacqua N, Campo M, Morassut M, Cecchini F. Grape Seeds: Chromatographic Profile of Fatty Acids and Phenolic Compounds and Qualitative Analysis by FTIR-ATR Spectroscopy. Foods. 2019;9(1):10.

- Santos CMM, Silva AMS. Nuclear Magnetic Resonance Spectroscopy for Structural Characterization of Bioactive Compounds. Comprehensive Analytical Chemistry, Elsevier. 2014;65:149-91.

- Pandya A, Howard MJ, Zloh M, Dalby PA. An evaluation of the potential of NMR spectroscopy and computational modelling methods to inform biopharmaceutical formulations. Pharmaceutics. 2018;10(4):1-24.

- Stockman BJ, Dalvit C. NMR screening techniques in drug discovery and drug design. Prog Nucl Magn Reson Spectrosc. 2002;41:187-231.

- Dalvit C. Ligand and substrate based 19F NMR screening: Principles and applications to drug discovery. Prog Nucl Magn Reson Spectrosc. 2007;51(4):243-71.

- Altemimi A, Lakhssassi N, Baharlouei A, Watson DG, Lightfoot DA. Phytochemicals: Extraction, Isolation, and Identification of Bioactive Compounds from Plant Extracts. Plants (Basel). 2017;6(4):42.

- Villa-Ruano N, Pérez-Hernández N, Zepeda-Vallejo LG, Quiroz-Acosta T, Mendieta-Moctezuma A, Montoya-García C, García-Nava ML, Becerra-Martínez E. 1H-NMR based metabolomics profiling of citrus juices produced in Veracruz, Mexico. Chem Biodivers. 2019;16:e1800479.

- Cérantola S, Breton F, Gall EA, Deslandes E. Co‐occurrence and antioxidant activities of fucol and fucophlorethol classes of polymeric phenols in Fucus spiralis. Bot Mar. 2006;49(4):347‐51.

- Jégou C, Kervarec N, Cérantola S, Bihannic I, Stiger‐Pouvreau V. NMR use to quantify phlorotannins: the case of Cystoseira tamariscifolia, a phloroglucinol‐producing brown macroalga in Brittany (France). Talanta. 2015;135:1‐6.

- Jégou C, Culioli G, Kervarec N, Simon G, Stiger‐Pouvreau V. LC/ESIMSn and 1H HR‐MAS NMR analytical methods as useful taxonomical tools within the genus Cystoseira C. Agardh (Fucales; Phaeophyceae). Talanta. 2010;83(2):613‐22.

- Cicero N, Corsaro C, Salvo A, Vasi S, Giofré SV, Ferrantelli V, Stefano VD, Mallamace D, Dugo G. The metabolic profile of lemon juice by proton HRMAS NMR: The case of the PGI interdonato lemon of Messina. Nat Prod Res 2015;29:1894–902.

- Kim H, Choi Y, Verpoorte R. NMR-based metabolomic analysis of plants. Nat Protoc. 2010;5:536–49.

- Selegato DM, Pilon AC, Carnevale Neto F. Plant Metabolomics Using NMR Spectroscopy. Methods Mol Biol. 2019;2037:345-362.

- Sengupta A, Weljie AM. NMR Spectroscopy-Based Metabolic Profiling of Biospecimens. Curr Protoc Protein Sci. 2019;98(1):e98.

- Kumar D, Thakur K, Sharma S, Kumar S. NMR for metabolomics studies of Crataegus rhipidophylla Gand. Anal Bioanal Chem. 2019;411(10):2149-59.

- Agregán R, Munekata PES, Franco D, Dominguez R, Carballo J, Lorenzo JM. Phenolic compounds from three brown seaweed species using LCDAD‐ ESI‐MS/MS. Food Res Int. 2017;99:979‐85.

- Heffernan N, Brunton NP, FitzGerald RJ, Smyth TJ. Profiling of the molecular weight and structural isomer abundance of macro-algae derived phloro-tannins. Mar Drugs. 2015;13(1):509‐528.

- Steevensz AJ, Mackinnon SL, Hankinson R, Craft C, Connan S, Stengel DB, Melanson JE. Profiling phlorotannins in brown macroalgae by liquid chromatography-high resolution mass spectrometry. Phytochem Anal. 2012;23(5):547-53.

- Zema DA, Calabrò PS, Folino A, Tamburino V, Zappia G, Zimbone SM. Valorisation of citrus processing waste: A review. Waste Manag Res. 2018;80:252-73.

- Prasain JK, Barnes S. Metabolism and bioavailability of flavonoids in chemoprevention: Current analytical strategies and future prospectus. Mol Pharm. 2007;4:846–64.

- Karas M, Bachmann D, Bahr U, Hillenkamp F. Matrix assisted ultraviolet laser desorption of nonvolatile compounds. Int J Mass Spectrom Ion Process. 1987;78:53–68.

- Hung W, Wang S, Chen Y, Yu H, Chen C, Yang W. Molecules. 2012;17:4950-61.

- Yao J, Utsunomiya S, Kajihara S, Tabata T, Aoshima K, Oda Y, Tanaka K. Mass Spectrom. 2014;3:1-10.

- Witkowska HE, Bialy Z, Jurzysta M, Waller GR. Nat Prod Com. 2008;3:1395- 410.

- Rivellino SR, Hantao LW, Risticevic S, Carasek E, Pawliszyn J, Augusto F. Food Chem. 2013;141:1828-33.

- Suzuki Y, Suzuki M, Ito E, Ishii H, Miseki K, Suzuki A. Glycoconj J. 2005;22:427-31.

- Ligor M, Ratiu I, Kiełbasa A, Al-Suod HH, Buszewski B. Extraction approaches used for the determination of biologically active compounds (cyclitols, polyphenols and saponins) isolated from plant material. Electrophoresis. 2018;39:1-15.

- Piccinelli AL, Mesa MG, Armenteros DM, Alfonso MA, Arevalo AC, Campone L, Rastrelli L. HPLC-PDA-MS and NMR characterization of C-glycosyl flavones in a hydroalcoholic extract of Citrus aurantifolia leaves with antiplatelet activity. J Agric Food Chem. 2008;56:1574–81.