An overview of basic structure and advancements of organic solar cells

- Manisha Bajpai1 C. K. Pandey2,* , Rakhee Malik3 and Abhishek Nath Mishra4

1,4 Department of Physics, Siddhartha University, Kapilvastu, Siddharth Nagar 272202, UP, India

2Department of physics, S.N.S College, Motihari 845401, BRABU, Muzafferpur Bihar, India

3Department of Physics, Government Degree College, Nadabhood Badaun 243723

Abstract

Recent scientific and economic research on organic solar cell has been developed during the last three decades due to their increase in power conversion efficiencies. This is achieved by the device engineering by introduction of new materials or tuning of materials used. This may lead towards sophisticated device structure of the device. Hence, we review here the different possible structure and how their mechanism changed by choosing different structures.

Keywords:  Organic solar cell, tuning of materials, photovoltaic device, organic materials, sophisticated device structure.

chanchalpandey44@gmail.com

- Introduction

Organic solar cells (OSCs) are  a type of photovoltaic device that convert sunlight into  electrical energy using organic materials. Unlike traditional silicon-based solar cells, which rely on inorganic semiconductors, OSCs utilize organic compounds as the active layer to absorb light and generate electric current. The key component of an OSC is the active layer, which typically consists of a blend of organic molecules or polymers. These materials have unique electronic properties that allow them to efficiently absorb photons across a wide range of wavelengths. When light strikes the active layer, it excites the organic molecules, creating electron-hole pairs. The separated charges, namely electrons and holes, are then transported through separate pathways within the organic layer to the respective electrodes, usually made of transparent conductive materials. The electrons flow towards the cathode, while the holes move towards the anode, creating an electric current. One of the advantages of OSCs is their potential for low-cost and large-scale production. Organic materials can be synthesized using relatively simple and inexpensive techniques, such as solution processing or printing methods, which opens up possibilities for flexible and lightweight solar panels.

However, OSCs currently face challenges in terms of efficiency and stability compared to their silicon counterparts. Efforts are being made to improve the performance of OSCs by developing new organic materials with enhanced light-absorbing and charge- transporting properties, as well as exploring novel device architectures and encapsulation techniques to enhance stability. Despite the current limitations, organic solar cells hold promise for various applications, including portable electronics, wearable devices, and building- integrated photovoltaics. Ongoing research and development in the field aim to overcome the existing barriers and unlock the full potential of OSCs as a renewable energy technology.

2. Basic structure

The basic structure of an organic solar cell typically consists of the following components:

2.1 Substrate:  A transparent material, such as glass or plastic, that serves as the foundation for the solar cell.

2.2 Transparent Conductive Layer: Usually made of indium tin oxide (ITO) or a similar material, this layer allows light to pass through while providing electrical conductivity.

2.3 Active Layer: This is the key component where the conversion of sunlight into electricity occurs. It consists of a thin film of organic semiconducting material, typically a polymer or a small molecule, which absorbs light and generates electron-hole pairs.

2.4 Electron Transport Layer: Located on top of the active layer, this layer facilitates the movement of electrons towards the electrode. Common materials used for this purpose include metal oxides like titanium dioxide (TiO2) or fullerene derivatives.

2.5 Hole Transport Layer: Positioned beneath the active layer, this layer assists in transporting the positively charged holes generated by the absorbed light towards the electrode. Materials such as PEDOT:PSS (poly(3,4-ethylenedioxythiophene) polystyrene sulfonate) are commonly used.

2.6 Back Electrode: This layer completes the solar cell structure and serves as the contact for collecting the generated electrons or holes. It is typically made of a metal such as aluminum, silver, or gold.

The incoming sunlight passes through the transparent layers and is absorbed by the active layer, causing the generation of electron-hole pairs. These charge carriers then move through their respective transport layers towards the back electrode, where they are collected and used to generate electric current.

# 3. Different layer and their use

## 3.1 Single layer

A single-layer organic solar cell, also known as a bulk heterojunction solar cell, is a type of photovoltaic device that converts sunlight into electricity using organic materials. Unlike traditional silicon-based solar cells, which typically consist of multiple layers, single-layer organic solar cells consist of a single layer of an organic semiconductor material. The key components of a single-layer organic solar cell are the active layer and the electrodes. The active layer is made up of a blend of two organic materials: a donor material and an acceptor material. The donor material, usually a polymer or small molecule, has the ability to donate electrons when it absorbs light, while the acceptor material has a high electron affinity and can accept the electrons from the donor material.  When sunlight strikes the active layer, photons of light excite the electrons in the donor material, causing them to be released and move towards the interface between the donor and acceptor materials. This creates separated electron-hole pairs, also known as excitons. The electrons then migrate through the donor and acceptor materials, generating an electric current. The electrodes, typically made of transparent conductive materials like indium tin oxide (ITO), are used to collect the generated current.

The advantage of single-layer organic solar cells lies in their simplicity and low-cost manufacturing potential. The use of organic materials allows for flexible and lightweight solar cell designs, which can be suitable for a wide range of applications. However, single-layer organic solar cells generally have lower efficiency compared to their multi-layer counterparts, which can limit their commercial viability. Ongoing research aims to improve the efficiency and stability of single-layer organic solar cells to make them more competitive with other solar cell technologies.

3.2 Double layer

A double-layer organic solar cell is a type of photovoltaic device that converts sunlight into electricity using organic materials. It consists of our two distinct layers of organic semiconductors with different electronic properties. The first layer is called the donor layer, which absorbs photons from sunlight and generates electron-hole pairs (excitons). This layer typically contains a polymer or small molecule material that has a high absorption coefficient for sunlight. Examples include polythiophenes, phthalocyanines, and fullerene derivatives. The second layer is known as the acceptor layer or the electron acceptor. It has a different electronic structure compared to the donor layer and is responsible for efficiently separating and transporting the generated charges. Fullerene derivatives, such as  PCBM (phenyl-C61-butyric acid methyl ester), are commonly used as acceptor materials due to their excellent electron mobility and compatibility with donor materials.

The interface between the donor and acceptor layers plays a crucial role in the device's performance. It is essential to ensure efficient exciton dissociation at this interface to separate the electron and hole pairs. To enhance exciton dissociation, an interfacial layer called the exciton-blocking layer or charge-blocking layer is often introduced. This layer prevents recombination of excitons and facilitates charge extraction.

The separated charges (electrons and holes) then move through their respective pathways within the cell. Electrons migrate through the acceptor material, while holes migrate through the donor material. Both pathways lead to their respective electrodes, creating a current flow.

To collect the generated current, transparent conducting electrodes, such as indium tin oxide (ITO) are typically used on both sides of the device. These electrodes allow light to pass through while enabling the extraction of the generated charges. In addition to the active layers and electrodes, a double-layer organic solar cell also includes other essential components. These include a hole transport layer (HTL) and an electron transport layer (ETL). The HTL assists in hole extraction and transport, while the ETL facilitates electron extraction and transport to the electrodes.

Overall, the double-layer organic solar cell operates based on the principles of photon absorption, exciton dissociation, charge transport, and charge collection. The efficient combination of organic materials with complementary electronic properties and proper device engineering are critical for achieving high power conversion efficiencies in these solar cells .

3.3 Active layer and their uses

In organic solar cells, the active layer is a crucial component responsible for converting sunlight into electricity. It typically consists of a blend or heterojunction of  organic materials with complementary electronic properties. The active layer plays a fundamental role in the absorption of photons, charge generation, and charge transport processes. Here's a detailed breakdown of the active layer and its uses in organic solar cells

:

4. Material Composition

  - Donor Materials: These are typically conjugated polymers or small organic molecules that have a high tendency to donate electrons upon light absorption. They are designed to have a broad absorption spectrum, allowing them to capture photons from a wide range of wavelengths.

  - Acceptor Materials: These materials have a high electron affinity and are responsible for accepting electrons from the donor materials. Fullerenes, such as C60 or C70, and non- fullerene acceptors (NFAs) are commonly used in organic solar cells.

5. Basic Mechanism Involved in Organic Solar Cells

5.1 Light Absorption:-The active layer contains light-absorbing molecules that absorb photons from the incident sunlight. The donor materials, in particular, possess a conjugated structure that enables efficient light absorption over a wide range of energies.

The absorption of photons creates excitons, which are bound electron-hole pairs. The excitons need to be dissociated into free charges to generate an electric current.

5.2 Exciton Dissociation:-The interface between the donor and acceptor materials is critical for exciton dissociation. The heterojunction between the two materials forms an energy gradient that promotes charge separation. Excitons migrate to the donor-acceptor interface, where they can separate due to the energy offset between the highest occupied molecular orbital (HOMO) of the donor and the lowest unoccupied molecular orbital (LUMO) of the acceptor. Efficient exciton dissociation is crucial to prevent recombination and maximize the generation of free charges.

5.3 Charge Transport:-Once separated, the free electrons and holes need to reach their respective electrodes to produce an electric current. The donor and acceptor materials have different mobilities for charge transport. Generally, donor materials have higher hole mobility, while acceptor materials have higher electron mobility. Balanced electron and hole mobilities are desirable to facilitate efficient charge transport and minimize losses due to trapping or recombination.

In summary, the active layer in organic solar cells consists of a blend or heterojunction of donor and acceptor materials. Its primary functions include light absorption, exciton dissociation, and charge transport. Optimizing the material composition, thickness, and morphology of the active layer is crucial for achieving efficient energy conversion in organic solar cells .

# 6. Factor affecting to device efficiency

Several factors can affect the efficiency of inorganic solar cells. Here are the key factors to consider:

6.1 Thickness and Morphology:-The thickness and morphology of the active layer are critical parameters that influence the device performance.

Optimizing the active layer thickness ensures a balance between light absorption and charge collection. If the layer is too thick, excessive photon absorption can lead to self-absorption or increased charge recombination. Conversely, if it is too thin, light absorption may be insufficient.

The morphology of the active layer, including the donor-acceptor nanoscale phase separation, affects exciton diffusion and charge transport. Controlled morphology can enhance the exciton dissociation efficiency and reduce charge carrier recombination.

6.2 Material Selection: The choice of materials plays a crucial role in determining the efficiency of inorganic solar cells. The properties of the materials, such as their bandgap, electron mobility, and light absorption capabilities, impact the efficiency of converting sunlight into electricity.

6.2.1 Bandgap: The bandgap of the semiconductor material used in the solar cell determines the portion of the solar spectrum that can be absorbed. The bandgap should be matched to the solar spectrum to maximize light absorption and minimize energy loss.

6.2.2 Light Absorption: Efficient light absorption is critical for high-performance solar cells. The materials used should have a high absorption coefficient and cover a broad range of the solar spectrum to capture as much sunlight as possible.

6.2.3 Charge Carrier Mobility: The mobility of charge carriers (electrons and holes) within the semiconductor material affects how effectively they can be collected and transported to the electrodes. Higher mobility reduces recombination losses and improves overall cell efficiency.

6.2.4 Interface Quality: The quality of the interfaces between different layers within the solar cell, such as the interface between the active layer and the electrodes, affects charge extraction and reduces losses due to recombination. Optimizing interface properties, such as surface passivation, can enhance the efficiency.

6.2.5 Energy Loss Mechanisms: Various energy loss mechanisms, such as recombination, transmission losses, and thermalization of carriers, can reduce the efficiency of solar cells. Minimizing these losses through material design and device engineering is essential to improve efficiency.

6.2.6 Light Trapping: Light trapping techniques, such as texturing the surface or using optical coatings, can enhance the absorption of light within the active layer, increasing the overall efficiency of the solar cell.

6.2.7 .Temperature Dependence: The performance of inorganic solar cells can be influenced by temperature. Higher temperatures can lead to increased carrier recombination rates, reducing the overall efficiency. Thermal management techniques are employed to mitigate this effect.

6.2.8 Device Architecture: The design and configuration of the solar cell structure impact its efficiency. Different architectures, such as single- junction, multi-junction, tandem cells, and nanostructured devices, offer unique advantages and challenges that can influence overall efficiency.

6.2.9 Manufacturing Processes: The fabrication processes used to manufacture solar cells, including deposition techniques, annealing conditions, and quality control, can affect the efficiency. Consistent and precise manufacturing processes are necessary for high-performance devices.

Optimizing these factors requires a deep understanding of the underlying physics, materials science, and device engineering to improve the efficiency of inorganic solar cells. Continuous research and development efforts are focused on addressing these factors to enhance solar cell performanc

7. Recent research on organic solar cell

Recent research on organic solar cells has focused on improving their efficiency, stability, and scalability. Scientists have been exploring new materials and device architectures to enhance the performance of organic solar cells. One area of research involves developing novel organic materials with improved light absorption properties and charge transport capabilities. This includes designing molecules with optimized energy levels and chemical structures to achieve higher power conversion efficiencies. Researchers are also working on improving the stability of organic solar cells to ensure their long-term performance. This involves developing materials that are less prone to degradation from environmental factors such as moisture, oxygen, and light exposure. Furthermore, efforts are being made to enhance the scalability of organic solar cells. Scientists are exploring large-area manufacturing techniques such as roll-to-roll printing and solution processing methods to enable cost-effective and high-throughput production of organic solar cell devices.  Additionally, advancements in device architectures, such as tandem or multi-junction structures, are being explored to further enhance the efficiency of organic solar cells. These architectures involve stacking multiple layers of different materials to capture a broader range of the solar spectrum and increase the overall power conversion efficiency.

Overall, recent research on organic solar cells aims to overcome the challenges associated with their efficiency, stability, and scalability, with a focus on developing new materials, improving device architectures, and advancing manufacturing techniques.

8. Advantages of Organic Solar Cells:  Organic solar cells, also known as organic photovoltaics (OPVs), have emerged as a promising renewable energy technology. In summary, the conclusion regarding organic solar cells can be stated as follows:

- Efficiency: Organic solar cells have made significant progress in terms of power conversion efficiency (PCE). Over the years, researchers have achieved PCE values exceeding 18%, closing the gap with traditional silicon-based solar cells. This advancement demonstrates the potential of organic solar cells as a viable alternative for clean energy generation.

- Flexibility: One of the key advantages of organic solar cells is their flexibility. They can be fabricated on flexible substrates, enabling applications in various forms such as curved surfaces, lightweight devices, and integrated into building materials. This flexibility opens up opportunities for new applications and designs,	expanding	the	scope of solar energy utilization.

- Cost-Effectiveness: Organic solar cells offer the advantage of being potentially cost-effective. They can be manufactured using low-cost, solution- based processes, such as printing or coating techniques, reducing production expenses. Additionally, the use of abundant and readily available organic materials further contributes to their cost competitiveness.

- Environmental Sustainability: Organic solar cells have a lower environmental impact compared to traditional solar technologies. The production of organic photovoltaics involves fewer energy-intensive processes and less hazardous waste generation. The use of organic materials also offers the potential for recyclability and end-of-life disposal, aligning with sustainable development objectives.

- Stability and Lifespan: While the stability and lifespan of organic solar cells have improved over time, they still face challenges in this regard. Organic materials can be susceptible to degradation over time due to environmental factors, such as moisture and UV radiation. Extensive research efforts are focused on enhancing the stability of organic solar cells to ensure longer operational lifespans.

- Market Adoption and Integration: Organic solar cells are gradually gaining attention and market adoption. Their unique characteristics, including flexibility, lightweight nature, and aesthetic appeal, make them suitable for various applications, such as wearable electronics, portable devices, and building- integrated photovoltaics (BIPV). Ongoing research and development efforts aim to further improve the performance, durability, and scalability of organic solar cells for broader deployment.

Conclusion: In conclusion, organic solar cells offer promising prospects for renewable energy generation. Continued advancements in efficiency, flexibility, cost-effectiveness, and environmental sustainability will contribute to their wider adoption and integration into various applications, thereby fostering a more sustainable and clean energy future.

References

- Rongbo Wang, Meidouxue Han, Ya Wang, Juntao Zhao, Jiawei Zhang, Yi Ding, Ying Zhao, Xiaodan Zhang, Guofu Hou, Recent progress on efficient perovskite/organic tandem solar cells,Journal of Energy Chemistry,Volume 83,2023,Pages 158-172,ISSN 2095-4956, https://doi.org/10.1016/j.jechem.2023.04.036.

- Walia Binte Tarique, Ashraf Uddin,A review of progress and challenges in the research developments on organic solar cells,Materials Science in Semiconductor Processing, Volume 163,2023,107541,ISSN 1369-8001, https://doi.org/10.1016/j.mssp.2023.107541.

- Shengfan Wu, Ming Liu, Alex K.-Y. Jen,Prospects and challenges for perovskite-organic tandem solar cells, Joule,Volume 7, Issue 3,2023,Pages 484-502,ISSN 2542-4351,https://doi.org/10.1016/j.joule.2023.02.014.

- Lluis F. Marsal, José G. Sánchez, Alfonsina A.A. Torimtubun, Organic Solar Cells,Editor(s): A.S.M.A. Haseeb, Encyclopedia of Materials: Electronics,Academic Press,2023,Pages 118-138,ISBN 9780128197356,https://doi.org/10.1016/B978-0-12-819728-8.00074-7.

- Manisha Bajpai, C.K. Pandey, Ritu Srivastava, Ravindra Dhar, Electrical transport properties of PFO: MEH-PPV polymer blends,Materials Letters,Volume 331,2023,133400,ISSN 0167-577X,https://doi.org/10.1016/j.matlet.2022.133400.