BACTERIOCIN BASED ACTIVE FOOD PACKAGING IN NANOTECHNOLOGICAL  PERSPECTIVE – AN OVERVIEW

JAYACHITRA.J1, P.SIVASAKTHIVELAN1   E.BABU2 and R.VAIDEESHWARAN1

1-Department of Agricultural Microbiology, Faculty of Agriculture,

Annamalai University, Annamalai Nagar, Tamil Nadu, India-608002.

2 -Lecturer, Department of Polymer Technology, Tamil Nadu

Government Polytechnic College Madurai, Tamil Nadu – 625 011.

Corresponding Author: microjayachitra@yahoo.com

ABSTRACT

Bacteriocins are antimicrobial peptides or proteinaceous materials produced by bacteria against pathogens. These molecules have high efficiency and specificity and are equipped with many properties useful in food-related applications, such as food preservatives and additives, as well as biomedical applications, such as serving as alternatives to current antibacterial, antiviral, anticancer, and antibiofilm agents. Despite their advantages as alternative therapeutics over existing strategies, several limitations of bacteriocins, such as the high cost of isolation and purification, narrow spectrum of activity, low stability and solubility, and easy enzymatic degradation, need to be improved. This review addresses nanotechnological innovations to improve antimicrobial properties and the application of bacteriocins through active food packaging systems. In this sense, nanotechnological techniques are described as alternatives for the physicochemical stabilization of bacteriocins and to enhance their antimicrobial properties. The incorporation of nano encapsulated bacteriocins in active packaging systems needs to be explored to expand the potential in the food industries.

Key words: Bacteriocin, Lactic Acid bacteria, Nanotechnology and Food Packaging

INTRODUCTION

Bacteriocins are a group of ribosomally synthesized peptides that are secreted extracellularly by various gram-positive and gram-negative bacteria (Cotter et al., 2005), although a majority of bacteriocins reported are produced by the former, especially lactic acid bacteria (LAB) (Hassan et al., 2012). Extensive studies have been carried out on bacteriocins owing to their excellent antibacterial activity, which is closely associated with strain producing species. In addition, bacteriocins have garnered considerable research attention in the field of biomedicine owing to their Generally Recognized as Safe (GRAS) status and because they are safe for human consumption due to their degradation by gastrointestinal proteases Silva et al., 2018. They are also being modified to improve the antibacterial spectrum. Most of the well-known bacteriocins are produced by gram-positive bacteria, where as only a few from gram-negative bacteria have been characterized (Ansari et al., 2015). The activities of these small bacteriocins consisting of cationic molecules (30–60 amino acids) vary throughout the antibacterial spectrum, mainly due to their amphiphilic helices. Bacteriocins are widely used as natural food preservatives substances that delay the growth of microorganisms in the food industry, because they are easily degraded by enzymes produced in the human gastrointestinal tract. The high quality and safety profile of bacteriocins as natural food preservatives is possible without the use of chemical preservatives, which is strictly regulated by governmental agencies, such as the Food and Drug Administration (FDA) in the United States, owing to their safety issues. Generally, bacteriocins can be directly added to food or incorporated into food during cultivation with the help of bacteriocin-producing bacterial strains.

Commercially bacteriocins permitted by the Food and Drug Administration (FDA) for use as biopreservatives are nisin and pediocin PA-1. These peptides can be added to food by inserting (i) the purified bacteriocin, (ii) the initial culture of the producing microorganism, or (iii) fermentation containing the bacteriocin. Nisin and pediocin have an antimicrobial action spectrum limited to gram-positive bacteria. Besides, they may exhibit instability in acidic and basic mediums and reduce the solubility over a wide pH range 3.4 Furthermore, bacteriocins are sensitive to proteolytic enzymes. Because of this, bacteriocins lose their antimicrobial activity due to the breaking of peptide bonds by proteases. Thus, new systems to improve their stability is promising research. The use of nanotechnology becomes an innovative alternative for bacteriocins to enhance their stability when applied to food.

Active packaging can be developed with antimicrobial compounds which interact with the food and or with the product’s internal atmosphere. There is a reduction in deterioration, improvement in food quality, and promoting an increase in shelf life. Furthermore, active packaging can be applied as a barrier to oxygen, moisture, and others. The release of antimicrobial substances controls the growth of unwanted microorganisms in food. Antimicrobial packages can be (a) sachets containing antimicrobial compounds; (b) incorporation of antimicrobial agents into the polymer matrix; or (c) use of antimicrobial polymeric raw materials. Thus, bacteriocins nanoencapsulation for subsequent application as active packaging can expand the food protection property. As nanomaterials present differentiated physicochemical properties, they can contribute to thermal stability, mechanical resistance, biodegradability, and flexibility. Besides, nanoencapsulated bacteriocins can be used as absorbing systems and active release of compounds to promote a barrier to moisture and gas exchange. These properties can be given to packaging through emerging and innovative technologies, such as nanotechnology.

Nanotechnology is being applied to the development of antimicrobial packaging, containing different encapsulated compounds that are physically and chemically unstable. Examples of encapsulated compounds include metallic silver nanoparticles, zinc oxide (Ceballos 2021, gold Yadav et al., 2021), copper Sahoo G2017 and titanium oxide (Esmailzadeh, 2021). Compounds obtained from natural sources have also been studied. Salama (2020), such as essential garlic oil, biomass of Spirulina sp. (Kamkar et al., 2021). phycocyanin (Kuntzler et al., 2020), acai extract (Terra et al., 2021). However, there are few studies on nanoencapsulation of bacteriocins (Silva et al., 2017). Nisin and pediocin belong to classes I and II, respectively. Both are electro statically attracted by the cell’s phospholipid membrane, acting on its permeability, according to the specificity of each class. The high content of anionic lipids present in Gram-positive cells explains the specificity of these bacteriocins. Therefore, nanomaterials with active functions provided by bacteriocins become promising alternatives for the development of food packaging and need to be further explored and presented in the literature.

Nanomaterials Combined with Bacteriocins

Potential of Nanotechnology for the application of Bacteriocins in Active Packaging can be developed with antimicrobial compounds which interact with the food and/or with the product’s internal atmosphere.

There is a reduction in deterioration, improvement in food quality, and promoting an increase in shelf life. Furthermore, active packaging can be applied as a barrier to oxygen, moisture, and others (Almasi, et al., 2020). The release of antimicrobial substances controls the growth of unwanted microorganisms in food. Antimicrobial packages can be (a) sachets containing antimicrobial compounds; (b) incorporation of antimicrobial agents into the polymer matrix; or (c) use of antimicrobial polymeric raw materials. Thus, bacteriocin nanoencapsulation for subsequent application as active packaging can expand the food protection property (Almasi et al., 2020). As nanomaterials present differentiated physicochemical properties, they can contribute to thermal stability, mechanical resistance, biodegradability, and flexibility.  Besides, nanoencapsulated bacteriocins can be used as absorbing systems and active release of compounds to promote a barrier to moisture and gas exchange (Singh et al., 2017). These properties can be given to packaging through emerging and innovative technologies, such as nanotechnology. Nanotechnology studies and manipulates materials with dimensions at the nanoscale and, due to this, can produce packaging with different physicochemical properties. The differentiated physicochemical properties are related to the high relation between surface and volume and the increase in the number of reactive sites in the nanomaterial. Due to these properties, the use of nanotechnology methods has shown promising results in the yield and stabilization of bioactive compounds (Sahoo et al., 2021). Nanoencapsulation can protect bacteriocins against the inactivation of proteolytic and lipolytic enzymes, extending the shelf life of foods. Furthermore, nanoencapsulation of bacteriocins can result in the reduction of high dosages of the antimicrobial agent in foods.

Table – 1. Antimicrobial spectrum and bacteriocin stability

| Bacteriocin | Producer microorganism | Bacteriological activity | Stability | Instability | Reference |
| --- | --- | --- | --- | --- | --- |
| Bac23 | Lactobacillus plantarum PKLP5 | Gram-positives and Gram-negatives | pH (2–12) Temperature 20–100°C | Proteolytic enzymes (trypsin and proteinase-K) | Sidhu and Nehra K. 2021. |
| ZFM54 | Lactobacillus paracasei ZFM54 | Gram-positives and Gram-negatives | Acidic medium Thermostable (30min, 100°C) | Proteolytic enzymes (trypsin and proteinase K) Fungi | Ye et al., 2021 |
| AMYX6 | Bacillus amyloliquefaciens JDF-17 | Gram-negative | Heat (121°C, 15 min) Acid-base (pH 2–10) | Trypsin, Protease K, Papain and Pepsin | Xiang et al., 2021 |
| Plantacillin B21AG | Lactiplantibacillus plantarum B21 | Gram-positive | up until 80°C | Proteinase K Gram-negative | Golneshin et al., 2020 |
| Nisin | Lactobacillus lactis spp. Lactis | Gram-positive | Heat | Gram negatives Interacts with proteins, lipids and carbohydrates being inactivated Proteolytic enzymes | Ibarra-Sanchez et al., 2020 |
| Pediocin PA-1 | Pediococcus pentosaceus NCDC 273 | Gram-positive | Heat (90°C, 5–10 min) | Gram negatives Fungi Proteolytic enzymes | Ibarra-Sanchez et al., 2020 |

Nanotechnology Approaches for Encapsulation of Active Compounds

Nanotechnology systems have a controlled release of compounds. Compound nanoencapsulation processes differ for each type of nanostructures, such as nanoemulsions, nanofibers or nanocapsules. In addition to the food properties, the nanostructured system to be used is related to the properties of the encapsulated antibacterial agent and the materials used. The nanostructures have the potential for application in foods Table - 2

Table - 2. jNano structures and their potential for application in food

| Nanostructures | Benefit | Disadvantages | Potential application in food | Techniques | References |
| --- | --- | --- | --- | --- | --- |
| Nanoliposomes . | Encapsulation of hydrophilic and lipophilic molecules. Physicochemical stabilization of encapsulated compounds. Controlled release system. | Low capacity to encapsulate Active substances. Dispersion Of nanoliposomes during the storage period | Embedding in coating material. | Microfluidization, ultrasound and rotor-stator | Beltran et al., 2020 |
|  |  |  |  | Cold high-pressure homogenization | Bianchi et al., 2020 |
|  |  |  |  | Lipid film hydration | Bianchi et al., 2019 |
|  |  |  |  |  | Heckler et al., 2020 |
| Nanoemulsions | Increase solubility and permeability of active substances. Permeate on rough surfaces. | Need for high amounts of surfactant, which can change product characteristics such as texture, flavor or make the food unsuitable for consumption. | Direct mix, Spraying or soaking, Incorporation into coating material, Embedding in edible film | Spontaneous emulsification | Lina et al., 2020 |
|  |  |  |  | Ultra-high pressure homogenization | Xu et al., 2017 |
|  |  |  |  | Phase inversion composition | Li et al., 2021 |
|  |  |  |  | Emulsion inversion point Phase inversion temperature | Hien et al., 2021 |
| Nanofibers . | Use of low temperatures that do not degrade the active compounds. They can be obtained with different conformations, sizes and polymers | High initial cost. Environment needs to have high temperature and humidity control. | Pads or sachets. Incorporation of antimicrobial agents into the polymeric coating or edible matrix. | Electrospinning | Terra et al., 2021 |
|  |  |  |  | Dry spinning | Tian et al., 2017 |
|  |  |  |  | Wet spinning | Abe & Utsumi 2020 |
|  |  |  |  | Gel spinning | Abe & Utsumi 2020 |
|  |  |  |  | Melt spinning | Abe K, Utsumi |
| Nanospheres | Active compounds are dispersed or solubilized within the polymer matrix, allowing physicochemical stabilization. Controlled release of compounds. | Low ability to encapsulate lipophilic compounds. High initial cost. Environment needs to have high temperature and humidity control | Direct mixing. Spraying or dipping. Embedding in coating material. Embedding in Edible Film | Electrospray | Coppock et al., 2021 |
|  |  |  |  | High-pressure homogenization technology and emulsion–solvent evaporation | Zhang et al., 2021 |
| Nanocapsules | Allows different active compounds to be adsorbed or dissolved in the polymer matrix. | High initial cost. Environment needs to have high temperature and humidity control | Direct mixing. Spraying or dipping. Embedding in coating material. Embedding in edible film. | Electrospray | Rodrigues et al., 2019 |
|  |  |  |  | High-pressure homogenization technology | Cui et al., 2020 |
|  |  |  |  | Emulsification-diffusion | Galindo-Perez et al., 2017 |
|  |  |  |  | Nanoprecipitation | Ramos et al., 2021 |

NANOLIPOSOMES

Liposomes are spherical structures that can contain a lipid bilayer or multiple bilayers (Fig. 1), and are classified as unilamellar and multilamellar, respectively. The difference between unilamellar and multilamellar liposomes is the vesicle size, which can range from 20 nm to 167nm. The formation of vesicles occurs spontaneously due to the hydrophilic interaction of the polar portion of the phospholipid with water. Nanoliposomes have the same structure and constituents as liposomes. However, they are not formed spontaneously (Maja, et al., 2020). Nanoliposomes can be applied as carriers of bioactive substances, such as antimicrobials and antioxidants, promoting the physicochemical stabilization of the molecule. Nanoliposimes can be added directly to the polymeric film forming solution to participate in the packaging manufacturing. In this context, Cui et al., used chitosan and Artemisia Annua Oil (AAO) liposomes to control the growth of E. coli O157:H7. For the film preparation, agar and chitosan were dissolved in water and acetic acid, respectively. The liposomes were incorporated into this solution at 45°C. The prepared solution was stirred and degassed by ultrasonic to remove air bubbles. The authors concluded that this film presented a potential for use as active packaging since the incorporation of AAO liposomes had a bacteriostatic effect. Thus, nanoliposomes can carry out the controlled release of these compounds into the system (food), extending the shelf life of the product. Other studies also proved the potential of nanoliposomes for application in active packaging. Sarabandi and Jafari produced betanin nanoliposomes added to isolated films of whey protein and chitosan, with potential application as active packaging. The authors found a reduction in water vapor permeability from 7.38 g Pa-1s-1m-1 to 5.46g Pa-1s-1m-1. Furthermore, the films added with nanoliposomes showed antibacterial activity against Staphylococcus aureus (63.45%). (Hamadou et al., 2020) evaluated the oxidation of β-carotene when encapsulated in lipid nanoliposomes composed of marine phospholipids (MPL) and egg phosphatidylcholine. The greatest inhibition of lipid peroxidation was obtained in MPL nanovesicles with b-carotene (42.9 – 2.18%). Thus, it was found that MPL nanoliposomes can be used to extend the shelf life of foods. Furthermore, according Lopez-Polo et al., the combined use of nanoliposomes with edible coatings is an innovative approach to improving the physical properties of these coatings. Therefore, this combination contributes to increasing the applicability of nanoliposomes in food packaging systems, improving the shelf life of foods through the gradual and controlled release of the encapsulated compound from the polymer matrix.

Fig. 1. Structural characteristics of nanoliposomes.

NANOEMULSIONS

Nanoemulsion (Fig. 2) is the dispersion of two immiscible solutions stabilized by surfactants or surfactant systems. The oil in- water (O/W) nano emulsion corresponds to the system where the particles have their hydrophobic part facing inwards with the oil and the hydrophilic part facing outwards in contact with water. Water-in-oil (W/O) nanoemulsion has the hydrophobic part of the particles facing outwards with the oil and the hydrophilic part inwards along with the water. High-energy or low-energy processes can be used to produce nanoemulsions Amin  and  Das (2019). High energy methods include high-pressure homogenization, ultrasonic treatment, and micro fluidization. Low energy techniques use chemical energy stored in components such as spontaneous emulsification and phase inversion temperature (Naseema  et al., 2020).

Nanoemulsions have the potential for application in food production, due to the droplets being stable in gravity separation and in the aggregation of encapsulated components. Nanoencapsulated bioactives have potentiated action due to the large area of contact surface, thus providing greater bioavailability. (Aswathanarayan 2019, Moreira et al., 2019). Nanoemulsions can be used in food coating solutions such as fruits. Concerning their application in food packaging, nanoemulsions must be solidified. In this case, the active compounds are dispersed within a continuous phase constituted by a film-forming matrix. This procedure requires Bacteriocin.

Fig. 2. Microscopic representation of the structure of nano emulsions oil and the hydrophilic part facing outwards in contact with water. Water-in-oil (W/O) nanoemulsion has the hydrophobic part of the particles facing outwards with the oil and the hydrophilic part inwards along with the water. High-energy or low-energy processes can be used to produce nanoemulsions. High energy methods include high-pressure homogenization, ultrasonic treatment, and microfluidization. Low energy techniques use chemical energy stored in components such as spontaneous emulsification and phase inversion temperature. Nanoemulsions have the potential for application in food production, due to the droplets being stable in gravity separation and in the aggregation of encapsulated components. Nanoencapsulated bioactives have potentiated action due to the large area of contact surface, thus providing greater bioavailability. Nanoemulsions can be used in food coating solutions such as fruits. Concerning their application in food packaging, nanoemulsions must be solidified. In this case, the active compounds are dispersed within a continuous phase constituted by a film-forming matrix. This procedure requires adding an emulsifier and introducing energy through homogenization or other processes. Finally, casting the filmforming formulations at a controlled thickness is applied to obtain a uniformly dry layer. Xiong et al. evaluated the effect of oregano essential oil (OEO), encapsulated in resveratrol nanoemulsion (RES) and incorporated in pectin matrix (PEC), in the preservation of fresh pork loin. The results showed that OEO coatings on RES nanoemulsions significantly extended the shelf life of pork. The pH values and color variations were stable. The lipid and protein oxidation were minimized, maintaining the tenderness of the meat and inhibiting microbial growth. The authors found that the edible coating of biopolymer loaded with nanoemulsion had the potential to produce active packaging for fresh meat.

NANOFIBERS

The obtainment of nanofibers can occur by several methods, such as drawing, template synthesis, self-assembly, and phase separation. Nanofibers can be developed with bioactive compounds, producing porous materials with high surface area concerning volume. Compared to materials on a macro-scale, nanofibers have high surface functionality, tensile strength, and stiffness, depending on the polymer matrix. They can be used in the synthesis of multilayer films, favoring the improvement of the mechanical properties of the polymeric film. The diameter on a nanometric scale provides greater flexibility and malleability of the material. However, some nanotechnological methods have disadvantages, including: complex and costly preparation steps; complementary techniques for drying the materials; producing particles with a high distribution of diameters; the using extreme temperatures and chemical solvents. In this sense, the electrospinning technique has the potential to encapsulate cells through simple, one-step processing. The principle of this process is a high-voltage application in the polymer solution resulting in the formation of nanofibers through electrostatic repulsion of charges and solution stretching (Fig. 3). In addition, the technique has advantages, such as (i) the use of organic and inorganic solvents, (ii) it does not use high temperatures, (ii) it has a uniform size distribution of the nanofibers, promoting the stability of bioactive compounds with controlled release, and (iv) produces dry nanofibers without agglomerates. Due to these properties, nanofibers are promising materialsfor applications in food packaging, through the nanoencapsulation of different bioactive compounds. Nanofibers can be produced to manufacture components for application to the primary packaging, such as a label, sachet, or stickers, to promote improved stability and controlled release of bioactive compounds. The application of nanofibers in active packaging can be carried out internally to preserving food quality. In this sense, Yang et al. have developed nanofiber films loaded with pullulan/ethylcellulose-cinnamaldehyde (CA) with inhibition against gram-negative (Escherichia coli) and gram-positive (Staphylococcus aureus) bacteria. Therefore, the authors suggested the use of films composed of nanofibers with CA as a potential material for application in active (antimicrobial) food packaging.

Fig. 3. Schematic representation of the production of nanofibers using the electrospinning technique.

NANOSPHERES AND NANOCAPSULES

Nanocapsules (Fig. 4a) and nanospheres (Fig. 4b) differ according to composition and structural organization. Nanocapsules are composed of polymers that surround core. Therefore, there is nuclear differentiation. However, nanospheres that do not contain oil in their structure are composed of a polymer matrix, in which there is no differentiation of the nucleus, and are composed of a more uniform matrix. Compounds can be encapsulated, adsorbed, or dispersed in nanoparticles. Nanocapsules and nanospheres can be added directly to the synthesis of the packaging material. Another alternative is to incorporate them into the polymeric solution, adding them into smaller components (sachets, labels, others) for addition to food packaging. Several nanoparticles composed of different materials (including lipids, inorganic materials, natural and synthetic polymers) have been developed (Pereira et al., 2020). These differentiated nanoparticle systems resulted in delivery systems with different physicochemical properties, providing several applications in the pharmaceutical, biomedical, food, and other industries. There are different methods for the development of polymeric microparticles and nanoparticles, such as electrospraying (Jarai et al., 2017), spray dryer coacervation, and polymerization (Assis et al., 2017) .

The electrospraying technique, known as electrohydrodynamic atomization, is widely used for obtaining nanoparticles. As electrospinning, electrospraying is a procedure based on the application of electrical potential, generating an electric field between the capillary and the collector (Ibili et al., 2019). Nanospheres’ characteristics produced by electrospraying can differ due to parameters such as flow rate, electrostatic potential, distance from capillary to the collector, and solution parameters (viscosity, density, and concentration). Environmental factors, such as temperature and pressure will also affect the nanospheres’ properties. In this context, (Schmatz et al., 2019) developed polymeric nanoparticles to encapsulate phycocyanin through the electrospraying technique. The nanoencapsulation from electrospraying keeps the pigment’s antioxidant activity at high temperatures (up to 216 °C). These results showed the potential of this nanoencapsulated phycocyanin for future applications as a component in food formulation. Nanoencapsulated Bacteriocins for Potentiation as Active Component Bacteriocins have antibacterial activity in their isolated form. However, they have low molecular stability when exposed to foods with high protein and lipid contents, which does not contribute to microbiological stability for a long time (Schmatz et al., 2019). Bacteriocins, when added to a food matrix or polymer matrix, have some disadvantages, such as (i) enzymatic inactivation in the presence of proteolytic enzymes (ii) interaction with food/ polymer matrix ingredients, and (iii) neutralization of anionic charges. These disadvantages can be overcome through the nanoencapsulation of bacteriocins, in order to protect them from the other constituents of the medium. Nanoencapsulation allows for the gradual rel . At the nanoscale, materials have unique properties. The high surface area, the potential for biological activity, and the controlled release of compounds make nanotechnology a strategy to increase the stability of bacteriocins (Gedarawatte et al., 2020). Most of these nanoencapsulation systems are used for bioactive agents (Radaic et al .,2020). However, there are few Fig. 3. Schematic representation of the production of nanofibers using the electrospinning technique (Yousefi et al., 2019). Fig.4. Structural characteristics of (a) nanocapsules and (b) nanospheres.studies with nano encapsulated bacteriocins applied in food packaging, and this punctual deficit becomes a promising research alternative. Nisin was encapsulated with microemulsions containing essential oil of dittany. The zone of inhibition formed during the development of Bacillus cereus was superior compared to the free nisin solution.

In plates contaminated with Bacillus cereus (6.8 log cfu ml-1), encapsulated nisin promoted total inhibition of the microorganism, while free nisin did not present an inhibition zone. In another study, polysaccharide-coated liposomes were used for nanoencapsulation of lysozyme and nisin. Phosphatidylcholine liposomes were coated with pectin, with an average particle size of 77 nm and encapsulation efficiency of 77–87% (Chatzidak et al.,,2019). Co-encapsulation of lysozyme and nisin reduced the microbial count of Listeria monocytogenes in the whole milk by 3 log CFU ml-1 and 2 log CFU ml-1 after 4 and 10 hours, respectively. The data proved the controlled release of nisin and lysozyme. Nisin liposomes entrapped in phosphatidylcholine and embedded in gelatin and cellulose films also showed high activity against L. monocytogenes (Lopes et al., 2019). The phospholipid must have good interaction with the bacteriocin and not reduce its antimicrobial effect (Boelter et al., 2016). Therefore, liposomes are a promising source for encapsulation of bacteriocins for application in the active material, as they provide stability and protection against molecular degradation (Sidhu et al., 2019). Nanoencapsulation of bacteriocins can be performed by nanoparticles and promote a controlled release of the antimicrobial agent. In a study developed by (Lee et al., 2018) chitosan nanoparticles were added with nisin, through ionic interactions between the positively charged amino groups of chitosan and the negatively charged tripolyphosphate ions of nisin. Free nisin reduced 2.73 log CFU ml-1 of Staphylococcus aureus in orange juice, while nisin-encapsulated nanoparticles reduced it by 3.82 log CFU ml-1.98 In another study, nisin was added to amaranth protein isolate to produce pullulan nanofibers by electrospinning process (Lee et al.,  2018). The release of nisin encapsulated in pullulan nanofibers occurred faster within 12 hours of analysis, resulting in 81.4% in acetate buffer at pH 3.4 (apple juice simulator). Thus, the release gradually reduced until it reached a constant cumulative. Encapsulated nisin nanofibers showed bactericidal activity against Listeria monocytogenes inoculated in apple juice after 20 h. According to Cui et al., nanofibers containing nisin-loaded nanoparticles present activity against Listeria monocytogenes. Moreover, these nanostructures did not change the sensory characteristics of the product (Soto et al., 2019).

Thus, studies prove that nanoencapsulation methods contribute to protecting bacteriocins, enhancing their bacteriostatic effects. (Cui et al., 2017). Furthermore, nanoencapsulation controls the release of the active compound, allowing product stability and consequently improving food preservation.

N Tab Nanocapsules (Fig. 4a)                               Nanospheres (Fig. 4b)

Conclusion

Several studies are carried out to determine different bacteriocin-producing strains with a broad spectrum of antibacterial action. However, the application of bacteriocins in foods is limited due to the low physicochemical stability of the molecule. The use of innovative techniques, such as nanoencapsulation from nanoliposomes, nanoemulsions, nanoparticles, and nanofibers, become an alternative for bacteriocins to have greater physicochemical stability when applied to foods. Research in the literature reports nanotechniques applied to bacteriocins with direct application in the food matrix. This review article presents techniques applicable to bacteriocins with the potential for incorporation into active packaging. The mentioned techniques are applied to bacteriocins and show high potential for insertion in active packaging. Nanoliposomes and nanofibers stand out due to their low cost and improvements to the mechanical properties of the packaging, respectively. Besides, both contribute so that a single and high dosage of the compound does not occur, ensuring microbiological stability of the food for a long time. However, these technologies are few explored in the literature for the encapsulation of bacteriocin, especially concerning its application in active food packaging. Future perspectives of the application of nanotechnology in the encapsulation of bacteriocins demonstrate several advantages. The nanoencapsulation of bacteriocins can be used to extend the shelf-life of foods, increase the safety of industrialized foods, and produce more efficient nutricosmetics, pharmaceuticals, and biomedical materials. These applications will be a reality soon after more studies addresses the molecular stability and controlled release of encapsulated bacteriocin from nanotechnology.

References

- Abe K, Utsumi M. Wet spinning of cellulose nanofibers via gelation by alkaline

treatment. Cellulose 2020;27(17):10441-10446. https://doi.org/10.1007/

s10570-020-03462-6.

- Assis RQ, Lopes SM, Costa TMH, et al. 2017.Active biodegradable cassava starch films incorporated lycopene nanocapsules. Ind Crops Products;109:818- 827. https://doi.org/10.1016/j.indcrop.2017.09.043.

- Almasi H, Oskouie MJ, Saleh A. 2020.A review on techniques utilized for design of controlled release food active packaging. Crit Rev Food Sci Nutr;1-21.

- Ansari, A. 2015. Bacteriocin from LAB for medical and health applications. In Beneficial Microorganisms in Medical and Health Appli-cations. Microbiology Monographs; Liong, M.T., Ed.; Springer: Cham, Switzerland,; Volume 28, pp.199–221.

- Aswathanarayan JB, Vittal RR. 2019.Nanoemulsions and their potential   applications in food industry. Front Sustain Food Syst;3:1-21. https://doi.org 10.3389/fsufs.2019.00095

- Amin N, Das B.2019. Review on formulation and characterization of herbosome complex. Int J Curr Pharm Res 201911(4):42-46. https://doi.org/10.22159/

ijcpr.2019v11i4.34948

- Beltra´n JD, Ricaurte L, Estrada KB, et al. 2019.Effect of homogenization methods on the physical stability of nutrition grade nanoliposomes used for encapsulating high oleic palm oil. LWT 2020;118:108801. https://doi.org/10.1016/ j.lwt..108801

- Bianchi A, Velot E´, Kempf H, et al.,.2020. Nanoliposomes from agro-resources as promising delivery systems for chondrocytes. Int J Molecul Sci ;21(10).

https://doi.org/10.3390/ijms21103436.

- Beltra´n JD, Ricaurte L, Estrada KB, et al. 2019..Effect of homogenization methods on the physical stability of nutrition grade nanoliposomes used for encapsulating high oleic palm oil. LWT 2020;118:108801. https://doi.org/10.1016/j.lwt. 108801

- Boelter JF, Brandelli A. Innovative bionanocomposite films of edible proteins

containing liposome-encapsulated nisin and halloysite nanoclay. Colloids

Surfaces B: Biointerfaces 2016;145:740-747. https://doi.org/10.1016/

j.colsurfb.2016.05.080

11.Ceballos RL, Bilderling C, Guz L, et al. 2021.Effect of greenly synthetized

silvernanoparticles on the properties of active starch films obtained by extrusion

and compression molding. Carbohydr Polym;261:117871.

- Cui H, Yuan L, Li W, et al. 2017.Edible film incorporated with chitosan and Artemisia annua oil nanoliposomes for inactivation of Escherichia coli O157:H7 on cherry tomato. Int J Food Sci Technol;52:687-698. 10.1111/ijfs.13322.

- Cui J, Sun C, Wang A, et al.2020. Dual-functionalized pesticide nanocapsule delivery system with improved spreading behavior and enhanced bioactivity. Nanomaterials ;10(2):220. https://doi.org/10.3390/nano10020220.

- Cui H, Wu J, Li C, et al. 2017. Improving anti-listeria activity of cheese packaging via nanofiber containing nisin-loaded nanoparticles. LWT - Food Sci Technol ;81:233-242

- Cotter, P.D.; Hill, C.; Ross, R.P. 2005. Bacteriocins: Developing innate immunity for food. Nat. Rev. Microbiol., 3, 777–788.

- Chatzidaki MD, Balkiza F, Gad E, et al. Reverse micelles as nano-carriers of

nisin against foodborne pathogens. Part II: The case of essential oils. Food

Chem;278:415-423. https://doi.org/10.1016/j.foodchem.2018.11.078

- Coppock J, Waxter Q, Hannan J. High temperature measurements of levitated

gold nanospheres derived from gold suspensions. J Quantitat Spectrosc

Radiative Transfer 2021;270:107645. https://doi.org/10.1016/

j.jqsrt.2021.107645.

17. Duong VA, Nguyen TTL, Maeng HJ, et al. 2019.Nanostructured lipid carriers

containing ondansetron hydrochloride by cold high-pressure homogenization

method: Preparation, characterization, and pharmacokinetic evaluation. J

Drug Delivery Sci Technol ;53:101185. https://doi.org/10.1016/

j.jddst.2019.101185.

18. Esmailzadeh H, Sangpour P, Shahraz F, et al. CuO/LDPE nanocomposite for

active food packaging application: A comparative study of its antibacterial

activities with ZnO/LDPE nanocomposite. Polym Bullet 2021;78(3):1671-1682.

19.Espitia PJP, Fuenmayor CA, Otoni CG. 2019. Nanoemulsions:

Synthesis,characterization, and application in bio-based active food packaging.

Compr Rev Food Sci Food Safety;18:264-285.

20. Gedarawatte STG, Ravensdale JT, Al-Salami H, et al. 2021.Antimicrobial efficacy of nisin-loaded bacterial cellulose nanocrystals against selected meat spoilage lactic acid bacteria. Carbohydr Polym;251:117096. https://doi.org/

10.1016/j.carbpol.2020.117096.

21. Gopi S, Balakrishnan P. 2021.Liposomal nanostructures: Properties and

applications. In: Sabu Thomas and Preetha Balakrishnan (Eds.). Nanoscale

Processing: Micro and Nano Technologies;163-179. https://doi.org/

10.1016/B978-0-12-820569-3.00006-

22.Galindo-Pe´rez MJ, Quintanar-Guerrero D,Cornejo-Villegas M,et al.  2017.Optimization of the emulsification-diffusion method using ultrasound to

prepare nanocapsules of different food-core oils. LWT - Food Sci Technol

2018;87:333-341. https://doi.org/10.1016/j.lwt..09.008

23. Gor MC, Golneshin A, Van TTH, et al. 2020.Cloning and functional expression of a

food-grade circular bacteriocin, plantacyclin B21AG, in probiotic Lactobacillus

plantarum WCFS1. PLoS ONE;15:1-18. https://doi.org/10.1371/

journal.pone.0232806.

24. Gupta A, Badruddoza AZM, Doyle PS. 2017;.A general route for nanoemulsion

synthesis using low-energy methods at constant temperature.

Langmuir33(28):7118-7123.

25. Golneshin A, Gor MC, Williamson N, et al. 2020.Discovery and characterisation of

circular bacteriocin plantacyclin B21AG from Lactiplantibacillus plantarum B21. 26. Heliyon;6(8):e04715. https://doi.org/10.1016/j.heliyon.2020.e04715

27. Hien LTM, Dao DTA. Black pepper essential oil nanoemulsions formulation

using EPI and PIT methods. J Food Process Preserv 2021;45(3):e15216. https://

doi.org/10.1111/jfpp.15216.

28.Hassan, M.; Kjos, M.; Nes, I.F.; Diep, D.B.; Lotfipour, F. 2012.Natural antimicrobial peptides from bacteria: Characteristics and potential applications to fight against antibiotic resistance. J. Appl. Microbiol., 113, 723–736.

29.  Hamadou AH, Huang W-C, Xue C, et al.2020. Comparison of b-carotene loaded marine and egg phospholipids nanoliposomes. J Food Eng 2020;110055.

https://doi.org/10.1016/j.jfoodeng.2020.110055

30. Hassan H, Gomaa A, Subirade M, et al. 2019.Novel design for alginate/resistant

starch microcapsules controlling nisin release. Int J Biol Macromol 2020;153:

1186-1192. https://doi.org/10.1016/j.ijbiomac..10.248.

31. Ibili H, Dasdemir M.2019. Investigation of electrohydrodynamic atomization (electrospraying) parameters’ effect on formation of poly (lactic acid) nanoparticles. J Mater Sci ;54:14609-14623.

32. Ibarra-Sa´nchez LA, El-Haddad N, Mahmoud D, et al. 2020.Invited review: Advances in nisin use for preservation of dairy products. J Dairy Sci;103(3):2041-2052. https://doi.org/10.3168/jds.2019-17498

33. Jarai BM, Kolewe EL, Stillman ZS, et al. 2020.Polymeric nanoparticles. In: E. J. Chung, L. Leon, & C. Rinaldi (Eds.). Nanoparticles for Biomedical Applications: Fundamental Concepts, Biological Interactions and Clinical Applications 303-324. https://doi.org/10.1016/B978-0-12-816662-8.00018-7

34. Heckler C, Marques MSC, Cacciatore AF, et al. Thymol and carvacrol in

nanoliposomes: Characterization and a comparison with free counterparts

against planktonic and glass-adhered Salmonella. Lwt 2020;127:109382.

https://doi.org/10.1016/j.lwt.2020.109382

35. Kuntzler SG, Costa JAV, Brizio APDR, et al. 2020.Development of a colorimetric pH indicator using nanofibers containing Spirulina sp. LEB 18. Food Chem;32:126768.

36. Kamkar A, Molaee-aghaee E, Khanjari A, et al. 2021. Nanocomposite active packaging based on chitosan biopolymer loaded with nano-liposomal essential oil: Its characterizations and effects on microbial, and chemical properties of refrigerated chicken breast fillet. Int J Food Microbiol;342:

109071.

37. Kirtonia K, Salauddin M, Bharadwaj KK, et al. 2021. Bacteriocin: A new strategic antibiofilm agent in food industries. Biocatal Agric Biotechnol;36:

102141. https://doi.org/10.1016/j.bcab.2021.102141

38. Lopes LQS, Vaucher AR, Giongo JL, et al. 2019.Characterisation and anti-biofilm activity of glycerol monolaurate nanocapsules against Pseudomonas aeruginosa. Microb Pathogen 2019;130:178-185. https://doi.org/10.1016/ j.micpath..03.007

39. Lee EH, Khan I, Oh DH. 2018.Evaluation of the efficacy of nisin-loaded chitosan nanoparticles against foodborne pathogens in orange juice. J Food Sci Technol;55(3):1127-1133.

40.Lahiri D, Chakraborti S, Jasu A, et al. 2020.Production and purification of bacteriocin from Leuconostoc lactis SM 2 strain. Biocatal Agric Biotechnol ;30:101845. https://doi.org/10.1016/j.bcab.2020.101845

41.Li HW, Xiang,YZ, Zang M, et al. 2021.A novel bacteriocin from Lactobacillus salivarius against Staphylococcus aureus: Isolation, purification, identification, antibacterial and antibiofilm activity. LWT;140:110826.https://doi.org/10.1016/j.lwt.2020.110826

42.Li H, Lu H, Zhang Y, et al. 2021.Oil-in-water nanoemulsion with reversible charge prepared by the phase inversion composition method. J Mol Liquids 2021;336: 116174. https://doi.org/10.1016/j.molliq.2021.116174.

43.Lopes LQS, Vaucher AR, Giongo JL, et al. Characterisation and anti-biofilm

activity of glycerol monolaurate nanocapsules against Pseudomonas

aeruginosa. Microb Pathogen 2019;130:178-185. https://doi.org/10.1016/

j.micpath.2019.03.007

44.Lina EC, Tama DP, Nelly N, et al. 2020.Design of nanoemulsion of Tephrosia vogelii extract as botanical insecticide to control cabbage pest. IOP Conference Series: Earth and Environmental Science;583(1). https://doi.org/10.1088/

1755-1315/583/1/012024

45.PLopes NA, Pinilla CMB, Brandelli A. 2019. Antimicrobial activity of lysozyme-nisin  co-encapsulated in liposomes coated with polysaccharides. Food Hydrocoll 93:1-9. https://doi.org/10.1016/j.foodhyd.2019.02.009

46.Mel JU, Gupta S, Willner L, et al. 2021.Manipulating phospholipid vesicles at the nanoscale: A transformation from unilamellar to multilamellar by an n-Alkylpoly( ethylene oxide). Langmuir;37(7):2362-2375. https://doi.org/

10.1021/acs.langmuir.0c03302

47.Maja L, Zˇ eljko K, Mateja P. 2020.Sustainable technologies for liposome preparation.J Supercrit Fluids 2020;165. https://doi.org/10.1016/j.supflu..104984

Moreira JB, Morais MG, Morais EG, et al. 2018. Electrospun Polymeric Nanofibers in Food  Packaging. In A. M. Grumezescu, & A. M. Holban (Eds.). Handbook of Food Bioengineering, Impact of Nanoscience in the Food Industry;387- 417.

48. Moreira JB, Goularte PG, Morais MG, et al. 2019.Preparation of beta-carotene nanoemulsion and evaluation of stability at a long storage period. Food Sci Technol 2019;39(3):599-604. https://doi.org/10.1590/fst.31317.

49.Moreira JB, Kuntzler SG, Terra ALM, et al. 2020. Electrospun nanofbers fundamentals, food packaging technology, and safety. In: S. M. Rangappa, P.

Jyotishkumar, S. M. K.  Thiagamani, S. Krishnasamy, & S. Siengchin (Eds.), Food     Packaging: Advanced Materials, Technologies, and Innovations (1st ed.). CRC Press..

50.Moreira JB, Lim LT, Zavarezec ER, et al. 2019.Antioxidant ultrafine fibers developed with microalga compounds using a free surface electrospinning. Food Hydrocoll;93:131-136.

51.Naskar, A.; Lee, S.; Lee, Y.; Kim, S.; Kim, K.-s. 2020. A new nano-platform of erythromycin combined with Ag nano-particle ZnO nano-structure against methicillin-resistant Staphylococcus aureus. Pharmaceutics, 12, 841.

52.Naseema A, Kovooru L, Behera AK, et al. A critical review of synthesis

procedures, applications and future potential of nanoemulsions. Adv Colloid Interface Sci 2021;287:102318. https://doi.org/10.1016/j.cis.2020.102318

53.Prakash A, Vadivel V, Rubini D, et al. 2019. Antibacterial and antibiofilm activities of linalool nanoemulsions against Salmonella Typhimurium. Food Biosci; 28:57-65. https://doi.org/10.1016/j.fbio.2019.01.018

54.Radaic A, Jesus MB, Kapila YL. 2020.Bacterial anti-microbial peptides and nanosized drug delivery systems: The state of the art toward improved bacteriocins. J Controlled Release;321:100-118. https://doi.org/10.1016/j.jconrel.2020.02.001

55.Ramos R, Koh K, Gabryelczyk B, et al.2021. Nanocapsules produced by

nanoprecipitation of designed suckerin-silk fusion Proteins. ACS Macro Lett 2021;10(5):628-634. https://doi.org/10.1021/acsmacrolett.1c00171

56.Razavi S, Janfaza S, Tasnim N, et al. 2021.Nanomaterial-based encapsulation for controlled gastrointestinal delivery of viable probiotic bacte´ria. Nanoscale Adv;3:2699-2709.

57.Rodrigues RM, Ramos PE, Cerqueira MF, et al. 2020..Electrosprayed whey proteinbased nanocapsules for b-carotene encapsulation. Food Chem ;314:126157. https://doi.org/10.1016/j.foodchem.2019.126157ANTAGES

58.Pereira DCS, Teixeira NS, Zin IM, et al. 2020.Active and intelligent packaging:

Security, legal aspects and global Market. Brazilian J Dev;6:61766-61794. DOI:10.34117/bjdv6n8-164 d Their Potential for Application in FoUCTUBENEFITS

59.Silva, C.C.G.; Silva, S.P.M.; Ribeiro, S.C. 2018. Application of bacteriocins and protective cultures in dairy food preservation.Front. Microbiol., 9, 594.

60.Sidhu PK, Nehra K. 2021. Purification and characterization of bacteriocin Bac23 extracted from Lactobacillus plantarum PKLP5 and its interaction with silver nanoparticles for enhanced antimicrobial spectrum against food-borne

pathogens. Lwt;139:110546. https://doi.org/10.1016/j.lwt.2020.110546

61.Shin, J.M.; Gwak, J.W.; Kamarajan, P.; Fenno, J.C.; Rickard, A.H.; Kapila, Y.L. 016.Biomedical applications of nisin. J. Appl. Microbiol., 120, 1449–1465.

62.Salama HE, Aziz AMS. Optimized carboxymethyl cellulose and guanidinylated

chitosan enriched with titanium oxide nanoparticles of improved UV-barrier

properties for the active packaging of green bell pepper. Int J Biol Macromol

2020;165:1187-1197.

63.Sahoo G, Sarkar N, Swain SK. 2017. Antimicrobial properties of nano-gold imprinted starch bionanocomposites. Polymer-Plastics Technol Eng;56(3):334-345.

Sahoo M, Vishwakarma S, Panigrahi C, et al. 2021.Nanotechnology: Current applications and future scope in food. Food Front;2(1):3-22.

64.Soto KM, Herna´ndez-Iturriaga M, Loarca-Pin˜a G, et al. 2019.Antimicrobial effect of nisin electrospun amaranth: Pullulan nanofibers in apple juice and fresh

cheese. Int J Food Microbiol295:25-32.

65.Singh T, Shukla S, Kumar P, et al. 2017.Application of nanotechnology in food science: Perception and overview. Front Microbiol 8:1-7.

66.Schmatz DA, Mastrantonio DJS, Costa JAV, et al. 2019.Encapsulation of phycocyanin by electrospraying: A promising approach for the protection of sensitive compounds. Food Biopr Process :119:206-215. https://doi.org/10.1016/j.fbp.2019.07.008

67.Sidhu PK, Nehra K. 2019.Bacteriocin-nanoconjugates as emerging compounds for enhancing antimicrobial activity of bacteriocins. J King Saud Uni Sci;31:758-767. https://doi.org/10.1016/j.jksus.2017.12.007

68.Sarabandi K, Jafari SM.2020. Effect of chitosan coating on the properties of

nanoliposomes loaded with flaxseed-peptide fractions: Stability during spraydrying.Food Chem;310:125951. https://doi.org/10.1016/

69.Silva CKD, Mastrantonio DJDS, Costa JAV, et al. 2019.Innovative pH sensors developed from ultrafine fibers containing ac¸aı´ (Euterpe oleracea) extract.Food Chem 294:397-404.

70.Tian Q, Xu Z, Liu Y, et al. Dry spinning approach to continuous graphene fibers

with high toughness. Nanoscale 2017;9(34):12335-12342. https://doi.org/

10.1039/c7nr03895j.

71.Terra ALM, Moreira JB, Costa JAV, et al. 2021. Development of time-pH indicator nanofibers from natural pigments: An emerging processing technology to monitor the quality of foods. Lwt;142:111020. https://doi.org/10.1016/ j.lwt.2021.111020.

72.Tripepi M, Ferrauto G, Bennardi PO, et al.2020. Multilamellar LipoCEST agents

obtained from osmotic shrinkage of paramagnetically loaded giant

unilamellar vesicles (GUVs). Angewandte Chemie ;132(6):2299-2303.

https://doi.org/10.1002/ange.201912327.

73.Vilela C, Kurek M, Hayouka Z, et al. 2018.A concise guide to active agents for active food packaging. Trends Food Sci Technol;80:212-222.

74.Xiong Y, Li S, Warner RD, et al. 2020.Effect of oregano essential oil and resveratrol nanoemulsion loaded pectin edible coating on the preservation of pork loin in modified atmosphere packaging. Food Control;114:107226.

75.Xu J, Mukherjee D, Chang SKC. 2018.Physicochemical properties and storage

stability of soybean protein nanoemulsions prepared by ultra-high pressure

homogenization. Food Chem;240:1005-1013. https://doi.org/10.1016/

j.foodchem.2017.07.077

76.Xiang YZ, Zhang YM, Liu YY, et al. 2021. Purification, characterization, and

antibacterial and antibiofilm activity of a novel bacteriocin against Salmonella

enteritidis. Food Control 127:108110. https://doi.org/10.1016/

j.foodcont.2021.108110

77.Yousefi M, Ehsani A, Jafari SM. 2019.Lipid-based nano delivery of antimicrobials to control food-borne bacteria. Adv Colloid Int Sci 2019;270:263-277. https:// doi.org/10.1016/j.cis..07.005

78.Ye P, Wang J, Liu M, et al. 2021.Purification and characterization of a novel

bacteriocin from Lactobacillus paracasei ZFM54. LWT 2021;143:111125. https://doi.org/10.1016/j.lwt..111125.

79.Yadav S, Mehrotra GK, Dutta PK. 2020.Chitosan based ZnO nanoparticles loaded gallic-acid films for active food packaging. Food Chem 2021;334.

80.Zamani-Ghaleshahi A, Rajabzadeh Gh, Ezzatpanah H, et al. 2020.Biopolymer coated nanoliposome as enhanced carrier system of perilla oil. Food Biophys; 15:273-287. https://doi.org/10.1007/s11483-019-09621-y.

81.Zhang K, Zhao W, Liu Q, et al. 2021.A new magnetic melt spinning device for

patterned nanofiber. Sci Rep;11(1):8895. https://doi.org/10.1038/

s41598-021-88520-0